Atmospheric mercury in Andacollo, a copper- and gold mining district of Chile

Pablo Higueras1, Roberto Oyarzun2, Javier Lillo3, Jorge Oyarzún4, & Hugo Maturana5

1Departamento de Ingeniería Geológica y Minera, Escuela Universitaria Politécnica de Almadén, Universidad de Castilla-La Mancha, Plaza M. Meca 1, 13400 Almadén, Spain. E-mail: pablo.higueras@uclm.es

2Departamento de Cristalografía y Mineralogía, Facultad de Ciencias Geológicas, Universidad Complutense, 28040 Madrid, Spain. E-mail: oyarzun@geo.ucm.es

3Escuela Superior de Ciencias Experimentales y Tecnología, Universidad Rey Juan Carlos, Tulipán s/n, 28933 Móstoles (Madrid), Spain. E-mail: j.lillo@escet.urjc.es

4Departamento de Minas and CEAZA, Facultad de Ingeniería, Universidad de La Serena, Casilla 554, La Serena, Chile. E-mail: joyarzun@userena.cl

5Departamento de Minas, Facultad de Ingeniería, Universidad de La Serena, Casilla 554, La Serena, Chile. E-mail: hmaturan@userena.cl

Abstract: This work reports the first data on atmospheric mercury for the mining district of Andacollo (Chile), where gold has been mined almost continuously since the Spanish colonial time (16th century). The district was initially exploited for gold and latter on for both gold and copper. Outdated gold recovery techniques involving milling and amalgamation are the largest contamination source of mercury, and yielded concentrations of about 104-105 ng Hg m-3, although much higher mercury concentrations were detected near the mills.

Key words: Atmospheric mercury. Mining. Andacollo. Chile.

BACKGROUND

The Andacollo mining district (copper-gold) is located in the Coquimbo region of Chile (Fig. 1), which is one of the richest in terms of mineral deposits and mining activity (Oyarzun, 1996).

These mineral deposits are not only important in terms of number and size, but also in historical significance, since some of the deposits have been exploited since the Spanish Colonial time (16th century). The historical gold production of the district is related to small-scale (artisanal) operations in which gold was, and is still recovered within the “trapiches” (Fig. 2a). The trapiches crushers consume 0.8 kg Hg/day, and only 50-70% of this mercury is later recovered. At present there are 8 trapiches in Andacollo, with a total of 30 operating trapiches. Smaller, more primitive crushers or “marays”, are also used in the Andacollo district (Fig. 2b). A large-scale gold-mining operation (open-pit mining, cyanide heap-leaching) was set up in the early 1990s, but it is now closing down. The total production reached ~ 20 metric tons (t) Au from 1996-2000, indicating an average annual production of 5 t Au. In addition, large-scale copper mining (open pit mining, heap-leaching recovery) has been set up by Minera Carmen (total annual production: 20,000 t of Cu metal). A preliminary environmental study in the region has been carried out previously by Higueras (in press).

Fig. 1: Location map of Andacollo and other mining sites and districts in

the Coquimbo region of Chile

Fig. 2: Trapiche (a) and maray (b) crushers in the Andacollo district

OBJECTIVES

Due to the toxic nature of mercury, which has been used in mining operations in close proximity to the town of Andacollo with a population of 10,000 inhabitants, we decided to measure the extent of contamination via an atmospheric mercury survey, to establish the possible hazards derived from continued mercury-gold amalgamation techniques.

METHODS

The survey was carried out along the main roads near the town of Andacollo, and at the mining and metallurgical sites, using a LUMEX RA-915+ analyzer for continuous mercury measurements. The analytical procedure is based on Zeeman atomic absorption spectrometry with high frequency modulation of light polarization (ZAAS-HFM). Application of Zeeman background correction and a multipath analytical cell provide high selectivity and sensitivity of measurements.

FINDINGS

The results of the survey are shown in figure 3. Mercury concentrations vary in Andacollo around 10 ng Hg m-3, however, areas near operating trapiches and marays have extremely high concentrations of 10,000 or even 100,000 ng Hg m-3.

Fig. 3: Atmospheric mercury in the Andacollo district. Continued measurements, t: time in seconds. Itinerary from La Serena to Andacollo, to small milling operations (marays and trapiches), to Dayton mining area, and back to Andacollo.

CONCLUSIONS

The air in some parts of Andacollo is as much as 100 times higher than the air quality guidelines maximum concentration of 1,000 ng Hg m-3 (WHO Regional Office for Europe, 2000). The highest mercury concentrations (Fig. 3) are concentrated where environmental remediation measures could reduce the problem. Although mercury concentration in air are below the maximum tolerable value, the concentrations are up to 10 times higher than those observed in generally uncontaminated regions (in the order of 1.8-2 ng Hg m-3, e.g., Wängberg, 2001; Kellerhals, 2003).

Acknowledgements

Adolfo Galván and Angel Patón collaborated in the field work. This research was founded by Spanish “Oficina de Cooperación Internacional” and “Vicerrectorado de Relaciones Internacionales”, University of Castilla-La Mancha. This manuscript was significantly improved by comments from an Organisation Committee reviewer, and from Dr. John E. Gray (U.S.G.S.).

REFERENCES

Higueras, P., Oyarzun, R., Oyarzún, J., Maturana, H., Lillo, J. & Morata, D. (2004). Environmental assessment of copper-gold-mercury mining at the Andacollo and Punitaqui districts, northern Chile. Applied Geochemistry, in press.

Kellerhals, M., Beauchamp, S., Belzer, W., Blanchard, P., Froude, F., Harvey, B., McDonald, K., Pilote, M., Poissant, L., Puckett, K., Schroeder, B., Steffen, A. & Tordon, R. (2003). Temporal and spatial variability of total gaseous mercury in Canada: results from the Canadian Atmospheric Mercury Measurement Network (CAMNet); Atmospheric Environment 37, pp. 1003-1011.

Oyarzun, R., Ortega, L., Sierra, J., Lunar, R. & Oyarzún, J. (1996). The manto-type gold deposits of Andacollo (Chile) revisited: A model based on fluid inclusion and geological evidence. Economic Geology 91, pp. 1298-1309.

Wängberg, I., Munthe, J., Pirrone, N., Iverfeldt, Å., Bahlman, E., Costa, P.,  Ebinghaus, R., Feng, X., Ferrara, R., Gårdfeldt, K., Kock, H., Lanzillotta, E., Mamane, Y., Mas, F., Melamed, E., Osnat, Y., Prestbo, E., Sommar, J., Schmolke, S., Spain, G., Sprovieri, F. & Tunce, G. (2001). Atmospheric mercury distribution in Northern Europe and in the Mediterranean region. Atmospheric Environment 35, pp. 3019-3025.

WHO Regional Office for Europe. 2000. Air Quality Guidelines for Europe. Second Edition. WHO Regional Publications, European Series, no. 91, 9 pp.