Electrokinetic remediation of mercury contaminated soil

Pascal Suèr1, Thomas Lifvergren2 & Bert Allard3

1Swedish Geological Institute, Olaus Magnus väg 35, SE-581 91 Linköping, Sweden,

e-mail: pascal.suer@swedgeo.se

2 Sydkraft SAKAB AB, SE-692 85 Kumla, Sweden, e-mail: thomas.lifvergren@sydkraft.se

3Man-Technology-Environment Research Centre, Örebro University,

SE-701 82 Örebro, Sweden, e-mail: bert.allard@nat.oru.se

Abstract: Mercury was removed from a field-contaminated soil by a combination of redox/complexation processes (with iodine/iodide) and electrokinetic mobilization. Iodide added to the cathode compartment, in an electromigration set-up, was transported into the soil and oxidised to iodine near the anode. After five days, around 50% of the total mercury content had migrated to the anode compartment, and another 25% remained in the soil surrounding the anode. No volatile mercury was formed. Electromigration is the dominant transport process for the (charged) mercury iodide complex The combination of iodide as complexing agent and an electric field for physical mobilization could be developed to a new method for in-situ remediation of mercury contaminated soil.

Key words: electrokinetic remediation, oxidation, complex formation, iodide

Introduction

Mercury in contaminated soil often binds strongly to the soil constituents, particularly organic matter and reduced sulphur. This prevents the release and reduces the mobility of mercury in the pore water system. Complexing agents such as chloride and iodide may increase the concentration of mercury in the soil water by forming negatively charged mercuric complexes (HgCl42- and HgI42-), thereby keeping mercury in the soil solution (Lumsdon 1995; Wasay 1995; Lifvergren 2001).

Electro-remediation has been considered for in-situ application on metal contaminated fine-grained soils. Both chloride and iodide have previously been used as mobilizing agents during electrokinetic remediation experiments. Cox (1996) added elemental iodine to a laboratory-contaminated soil, since iodide alone was not sufficient to mobilize the mercury. The iodine would oxidize the mercury and thereby release it from the soil matrix. The released mercury would form a soluble iodide complex, which could be transported by the electric field.

In the present study, the combination of an electric field and addition of iodide was tested on a contaminated soil containing high levels of strongly bound mercury.

## Experimental

Soil was taken from the site of a chlor-alkali factory (Bohus, Sweden). The soil was a predominantly illitic clay with a loss on ignition of 2.5 % and a sulphur content of 4 g/kg. The mercury content was 80-90 mg/kg. The mercury was associated with organic matter and secondary sulphides and was highly immobile in this soil (Lifvergren 2001).

Homogenized soil (1.5 kg) was placed in an open plastic box between two polypropylene geotextile filters 20 cm apart (Fig. 1). Electrodes (a titanium cathode and an iridium-coated titanium anode) were placed in the electrode compartments. Water (2 litres) was distributed over the soil and the electrode compartments. The entire set-up was placed in a box. The air was continuously pumped out into 1 M nitric acid solution, where volatile elemental mercury would be trapped. Potassium iodide was added to the cathode compartment, and an electric field corresponding to a potential of 30 V was applied. Sampling took place after 1, 3 and 5 hours after start-up and thereafter every 12 hours up to 122 hours. Soil samples were taken at the end of the experiment.

Figure 1. Experimental set-up

## Results and discussion

Table 1 provides a summary of the conditions at the end of the experiment. Dissociation of water at the electrodes had the expected effect on pH, Fig. 2. In the cathode compartment the pH rose to above 12, and the hydroxide ions migrated into the soil. The initial concentration of iodide was 45 mM in the cathode compartment. The iodide migrated into the soil towards the anode, Fig. 3. After 122 hours, only 1.7 mM iodide remained in the cathode compartment. The total amount of iodide in the system decreased

Table 1: Summary of analyses after 122 hours

___________________________________________________________

Pore		Anode	      Distance from the anode (cm)	Cathode

Water			3.5	7.5	12.5	17.7	22

----------------------------------------------------------------------------------------------

pH		1.3	1.6	2.8	3.6	6.1	12.8	13.2

I- (mM)		0.7	3.0	1.5	1.1	0.7	1.1	1.7

Hg (mg/l)	163	51	8.8	1.1	0.3	0.1	0.2

Soil

Water (kg/kg)		0.41	0.48	0.39	0.47	0.48

Hg (mg/kg)		13	36	20	11	10

___________________________________________________________

Original Hg-concentration in the soil: 90 mg/kg

steadily during the experiment, and less than 5 % of the iodide remained in solution after 122 h. When the iodide ions moved towards the anode, they met an environment that was increasingly acidic and oxidizing leading to a formation of elemental iodine and tri-iodide ions. After the 122 hours of the experiment, only ca 1 mM iodide remained in the soil solution.

Figure 2: pH in the pore water			Figure 3: Iodide in the pore water

 0 h; + 3 h;  5 h; o 20 h;  68 h;  122 h	 0 h; + 3 h;  5 h; o 20 h;  68 h;  122 h

Figure 4: Mercury in the pore water		Figure 5: Mercury  in the soil matrix

 0 h; + 3 h;  5 h; o 20 h;  68 h;  122 h		 0 h;  122 h

Mercury concentrations in the soil water increased as the iodide ions proceeded into the soil, Fig 4. An iodide concentration of 0.3 mM was enough to start the mobilization of mercury, which then moved towards the anode. Thus, the dominant mercury species in the water phase was anionic, most likely HgI3- or HgI42-. The mercury distribution in the soil water was close to a steady state after 68 hours. The concentration of mercury in the anode compartment was then 190 mg/l. The mercury iodide complex was still migrating towards the anode, but the concentration in the anode solution was so high that diffusion back into the soil must be possible. Evidently, iodide was able to mobilize mercury in the present soil, which was contrary to the results by Cox (1996). The main differences between the two studies are the chemical state of the mercury and the over-all chemical conditions (including an excess of organics in the soil used by Cox).

Detailed discussions of the results are given elsewhere (Suèr 2001; Allard 2003).

Conclusions

The combination of electro-remediation and the addition of iodide lead to a removal of mercury from contaminated soil due to a combined redox and complexation process. Optimisation with respect to the amount of iodide is needed, as well as some improvements in the design.

## Acknowledgements

This research was supported by the Swedish Foundation for Strategic Environmental Research (Mistra), the Swedish Research Council (TFR), the SAKAB-Kumla Foundation,  Sydkraft SAKAB AB and the Knowledge Foundation.

## References

Allard, B., Lifvergren, T., Suèr, P. (2003): Metal-organics in soil. In COLDREM - Soil Remediation in Cold Climate. Synthesis Report, Mistra, Stockholm, pp. 104-111.

Cox, C. D., Shoesmith, M. A., Ghosh, M. M. (1996): Electrokinetic remediation of mercury-contaminated soils using iodine/iodide lixiviant. Environmental Science and Technology 30, pp. 1933-1938.

Lifvergren, T. (2001): Remediation of mercury polluted soil. Örebro Studies in Environmental Science 1, Örebro University (Diss).

Lumsdon, D.,G., Evans, L., J., Bolton, K. A. (1995): The influence of pH and chloride in the retention of Cd, Pb, Hg, and Zn by soils. Journal of Soil Contamination 4, pp. 137-150.

Suèr, P. (2001): Electroremediation of contaminated soil. Örebro Studies in Environmental Science 2, Örebro University (Diss).

Wasay, S., A., Arnfalk, P., Tokunaga, S. (1995): Remediation of a soil polluted by mercury with acidic potassium iodide. Journal of Hazardous Materials 44, pp. 93-102.