Mercury contamination in fish from gold mining areas in the Amazon Region and human health risk assessment

Zuleica C. Castilhos1, Saulo Rodrigues-Filho1, Ana Paula Rodrigues1, 3, Roberto C. Villas-Bôas1, Marcello M. Veiga2, Christian Beinhoff2.

1 CETEM, Centre for Mineral Technology, Av. Ipê, 900, 21941-590,  Rio de Janeiro, Brazil, zcastilhos@cetem.gov.br; 2 UNIDO, United Nation Industrial Development Organization, Vienna International Centre, P.O. Box 300, A-1400, Vienna, Austria; 3 Fluminense Federal University, Department of Geochemistry, Niterói, Brazil.

Abstract: The main objective of this work is to conduct an assessment of the magnitude of

mercury contamination in fish from the target Amazonian region influenced by gold mining areas and perform the human health hazard assessment due to fish consumption. It was investigated the mercury levels in fish from all 11 sites in two areas: 4 sites in São Chico and 7 in Creporizinho area. The results showed that total mercury in fish from São Chico (2.533.91g/g) is significantly higher than in fish from Creporizinho area (0.360.33g/g). The most contaminated sites are located in São Chico, São Chico reservoir and Buriti mining site, in Creporizinho. By employing the mercury risk assessment to human health due to fish consumption, the results suggest that possibly in all sites, except in reference area (Chico Chimango creek), populations are subject to potential hazards, being site A2 the most evident case of mercury pollution. Although the estimated by single compartment-model and verified blood and hair Hg concentrations in local population are lower than values (~200g.l-1 and 50 g.g-1) associated with adverse effects on nervous system, manifested as an approximately 5% increase in the incidence of paraesthesia, it is important to realize that there are some individuals with higher levels of mercury in blood and hair in São Chico area. The estimated blood and hair Hg concentration in Creporizinho fall closer to the observed.

Keywords: Amazonian fish, mercury, gold mining, human health hazard

Introduction

The present work describes part of the results achieved in two small scale gold mining areas in the Brazilian Amazon - São Chico and Creporizinho - as part of the environmental and health assessment (E&HA) conducted by the Centre for Mineral Technology (CETEM) with the colaboration of the Evandro Chagas Institute (IEC), under the general coordination of the United Nations Industrial Development Organization (UNIDO). The E&HA is a part of the GEF/UNIDP/UNIDO Global Mercury Project - Removal of Barriers to the Introduction of Cleaner Artisanal Gold Mining and Extraction Technologies.

The main objective of this work is to conduct an assessment of the magnitude of mercury contamination in fish from the target Amazonian region influenced by gold mining areas and perform the human health hazard assessment due to fish consumption. Fish sampling was conducted in August 2003, at two study areas, at São Chico and at Creporizinho mining sites or “garimpo” areas. Both areas are located inside of the Mineral Tapajós Reserve, State of Para, between the cities of Jacareacanga and Itaituba, where the mining sites are distributed alongside the tributaries of the Tapajós river. However, these two areas belong to two distinct hydrographic basins: Jamanxin river basin and Crepori river basin, respectively.

Results and Discussion

The São Chico mining site is located 5 km distance from the Transgarimpeira road. The village consists of 63 houses and 134 individuals, being 41% of garimpeiros. From the beginning of the very first gold mining activity in 1963, the village has shown two main periods of prosperity, one in the end of the 80´s, and other in the end of the 90´s, when gold rich primary deposits were discovered. Nowadays, exploitation of primary gold ore is over, being gold production in São Chico almost restricted to the reprocessing of tailings produced during the 80´s.

Creporizinho is a typical gold mining village with an estimated population of 1000 inhabitants.The mining sites of Papagaio, Areal, Tabocal and Bofe are located near the village, where alluvial gold has been explored, and from the middle of 90’s exploration of primary started. Nowadays, alluvial gold is rare, being mining of lateritic soil, primary ore and reworking of tailings common all over the area.

A total of 234 fish specimens of 16 species were collected: 73 specimens belonging to 13 species in São Chico and 161 specimens of 11 species in Creporizinho. A total of 7 common species could be collected in both areas (acari, cará, curimatã, mandi, piau, piranha and traíra). It was investigated the mercury levels in fish from all 11 sites belonging to two areas: 4 in São Chico (A1-flooded open pits; A2-São Chico reservoir, A3-flooded open pit near Rosa stream; A4- Conrado inflow and Novo river) and 7 in Creporizinho (A5-Papagaio mining site; A6- Tabocal mining site, A7-Buriti mining site; A9- Porto Alegre, Crepori river; A10- Crepori river upstream of Creporizinho inflow; A11- Chico Chimango creek, clear water near the Creproizinho inflow into the Crepori river). It should be stressed that was not easy fishing near the garimpo’s areas, mainly by high turbidity from mining activities, which has as a consequence, fish avoidance.

The results show that total mercury in fish from São Chico (2.533.91g/g, n=73) is higher than in fish from Creporizinho area (0.360.33 g/g, n=161; p<0.0001) and that the minimum values for Hg in fish are similar between areas (0.025-0.027g/g), but the maximum values are one order of magnitude higher in São Chico (21.90g/g) than in Creporizinho area (2.10g/g).

The mean concentration of Hg (1.04g/g) in fish species from this work was similar to that from species from some Amazonian rivers, mainly contaminated rivers (Akagi, 1994, Castilhos, 1998). However, São Chico area has shown Hg levels in fish that are considered abnormally high, comparing then with results from previous works. Among the analyzed fish, 35% of total sampled fish, belonging to 6 species showed Hg concentrations above 0.5 g/g, the limit for human protection by Hg exposure by fish consumption (WHO, 1990). Whereas in Creporizinho 22% of fish samples showed Hg levels above that limit, in São Chico this percentage increases to more than 60%.

The present results show that fish from São Chico are more contaminated, heavier and larger than those from Creporizinho area. However, no correlation was found between fish Hg concentration and weight or length when analyzing all specimens in both areas. Comparing mercury levels in fish from each site within each study area (São Chico and Creporizinho) the results showed that fish from the São Chico Reservoir (A2) present the highest mercury levels in São Chico and are smaller than fish from the other sites (A1, A3 and A4) (Table 1). In Creporizinho area, fish from the site A8 showed higher Hg levels than all the other sites, and the specimens are also smaller than fish from the most of the other sites (A6, A9, A10 and A11). However, the Hg levels in fish from A2, São Chico area, are close to 7 times higher than in A8, Creporizinho area.

Table 1 - Total Hg in fish (ww) from different sites of garimpo’s areas

| Garimpo area | N | Mercury (g/g) | N | Length (cm) | N | Weight (g) |
| --- | --- | --- | --- | --- | --- | --- |
| São Chico | 73 | 2.533.91 | 73 | 18.7514.42 | 32 | 934.31,681.7 |
| A1 | 15 | 0.570.75 | 15 | 12.334.01 | 1 |  |
| A2 | 33 | 4.974.80 | 33 | 13.116.83 | 13 | 200.081.64 |
| A3 | 16 | 0.320.15 | 16 | 22.0413.2 | 9 | 166.766.1 |
| A4 | 9 | 0.830.43 | 9 | 43.6120.0 | 9 | 2,838.92,294.5 |
| Creporizinho | 161 | 0.360.33 | 161 | 11.624.86 | 49 | 191.8186.0 |
| A5 | 17 | 0.270.16 | 17 | 9.531.88 | - |  |
| A6 | 15 | 0.300.30 | 15 | 13.47.15 | 5 | 290.0134.2 |
| A7 | 33 | 0.320.35 | 33 | 9.802.57 | 3 | 150.086.6 |
| A8 | 39 | 0.660.33 | 39 | 8.572.33 | 2 | 150.070.7 |
| A9 | 22 | 0.250.28 | 22 | 13.23.00 | 12 | 129.762.0 |
| A10 | 10 | 0.320.17 | 10 | 15.759.25 | 6 | 341.7196.4 |
| A11 | 25 | 0.130.10 | 25 | 16.063.58 | 21 | 171.448.9 |
| Total | 234 | 1.042.42 | 234 | 13.849.56 | 81 | 485.21,118.0 |

By employing the risk assessment to human health,  toxicological, rather than simply a statistical tool, significance of the contamination can be ascertained. At a screening level, a Hazard Quotient (HQ) approach (USEPA, 1989), assumes that there is a level of exposure (RfD = Reference of Dose) for non-carcinogenic substances below which it is unlikely for populations to experience adverse health effects. The MeHg RfD value is 1E-04 mg.Kg-1.d-1 (IRIS, 2003). The estimated exposure levels is the product of Hg concentration considering all fish and the adult human ingestion rate for local populations. In São Chico area, close to 60% of the population consume fish just once a week and 90% consume fish twice weekly, as well as for Creporizinho area (Armin, 2004). Thus, it has been considered reasonable a consumption rate close to 0.02 Kg.d-1 in both areas. HQ resulted for all study sites above the unity, from 1.5 to 28.5, except for A11, which is considered as a reference area. The São Chico reservoir showed the higher values and in Creporizinho area, the HQ are close to 2 for all sites, except A8 (Buriti site), where the HQ is 3.3. From these results, one could suggest that possibly in all sites, except in A11, and depending on their diet habits, populations are subject to potential hazards due to fish consumption, being site A2 the most evident case of mercury pollution.

Hg concentration in blood (C) in g.l-1 can be related to the average daily dietary intake (d) in µg of Hg, as follows: C = 0.95 * d. Hair concentrations of Hg are proportional to blood concentrations at the time of the formation of the hair strand, and blood-to-hair ratio in humans is about 1 to 250 (WHO, 1990). The estimates to Hg concentration in blood and in hair for São Chico and Creporizinho areas showed in São Chico 98.2 g.l-1 and  24.5 g.g-1, respectively. These data do not agree with the determined in the local population 21.6g.l-1 and 3.162.63 g.g-1 (n=136). However, there are some individuals with higher levels of mercury in blood (as 141g.l-1) and hair (15 g.g-1) in this area. Considering fish only from A1, A3 and A4, the estimated Hg in blood and hair resulted in 23.5g.l-1 and 5.9 g.g-1, respectively, which are closer to the verified ones. As a result, one could suggest that consumption of fish from A2 appears to be uncommon within the local community. This has been confirmed through interviews with the local population, probably as a result of a general perception towards Hg contamination in the reservoir (A2). On the other hand, it is highlighted that the estimated blood and hair Hg concentration in Creporizinho fall closer to the observed 23.2127.11 g.l-1 and 1.821.53 g.g-1  of total Hg concentration, respectively.

Conclusions

Considering the two gold mining areas studied and their sites, the resulted Hg levels in fish suggest that there are two most contaminated sites, São Chico reservoir (A2) in São Chico area and Buriti site (A8), in Creporizinho area. However, the Hg levels in fish from the first one is close to 7 times higher than in A8. For all sites, except in A11, and depending on their diet habits, populations are subject to potential hazards due to fish consumption. The site A2 showed the higher increase of potential human health hazard. Although both the estimated by single compartment-model and verified blood and hair Hg concentrations in local population are lower than values (~200g.l-1 and 50 g.g-1, respectively) associated with adverse effects on nervous system, manifested as an approximately 5% increase in the incidence of paraesthesia, it is important to realize that there are some individuals with higher levels of mercury in blood and hair in São Chico area. Contrasting, clinical observations in Iraq suggest that women are more sensitive to the toxic effects of MeHg during pregnancy, suggesting a greater risk for pregnant women and, especially for their offspring (WHO, 1991). It may be important, since close to 23% of the population, in both areas, are women within reproductive age. On the other hand, one could suggest that consumption of fish from A2 appears to be uncommon within the local community. It is important to note that communities from this gold mining areas are under potential hazard by Hg vapor exposure, but, unfortunately, Hg levels in atmosphere are not available at this time to estimate the Hg daily doses by inhalation. Finally, it should be emphasized for the local community the potential hazard to human health due to consumption of fish from that site, besides the potential hazard of inorganic mercury vapor by occupational exposure.

# References

Akagi, H., Kinjo,Y., Branches, F., Malm, O., Harada, M., Pfeiffer, W.C., Kato, H. (1994): Methylmercury pollution in Tapajos river basin, Amazon. Environ Sci; Vol. 3, pp. 25-32.

Castilhos, Z.C., Bidone, E.D., Lacerda, L.D. (1998): Increase of the reference human exposure to mercury through fish consumption due to gold mining at the Tapajos river region, Amazon. Bull. Environ. Contam. Toxicol.; Vol 6, pp. 202-209.

IRIS- International Agency for Cancer Research (2003): www.usepa.gov/iris

U.S.EPA. – United States Environmental Protection Agency (1989): Risk Assessment Guidance for Superfund. Vol I: Human Health Evaluation Manual.  Washington DC.

WHO (1990): Environmental Health Criteria 101: Methylmercury. Geneva, World Health Organization.

WHO (1991): Environmental Health Criteria 118. Inorganic mercury. International Program on Chemical Safety, World Health Organization, Geneva, Switzerland.