Hg and Se contents in sediment of the Barra Bonita and Bariri Reservoirs (Medium Tietê River, SP – Brazil), and it availability.

Fragoso, D.D1.; Stolf, F.P.B2.; Moura, M.A.M3.; de Souza, S.S1; Mortatti, J1 and Ferreira, J.R4,1

1 Centro de Energia Nuclear na Agricultura – CENA-USP, Universidade de São Paulo. Av. Centenário 303, 13400-970 – Piracicaba, SP, Brasil; 2 Faculdade de Engenharia, Arquitetura e Urbanismo – Universidade Metodista de Piracicaba – UNIMEP, 13450-000 - Rodovia Santa Bárbara/Iracemápolis, Km 01 – Sta Bárbara D’Oeste, SP, Brasil; 3 Universidade Federal de São Carlos – UFSCAr; Rod. Washington Luiz, Km 235, 13565 – 905 – São Carlos, S.P, Brasil;  4 Instituto de Pesca – Agência Paulista de Tecnologia dos Agronegócios – APTA – Av. Francisco Matarazzo 455, 05001-900, São Paulo, SP, Brasil (ferreira@cena.usp.br).

Abstract: The purpose of this paper was to assess the levels of Hg in the Barra Bonita and Bariri Reservoirs, because in 1981, high concentrations of total Hg were found in carnivorous fishes (plagioscion squamossis), from the Barra Bonita Reservoir. In addition, data on Se biogeochemistry, due to its antaghonic relationship with Hg in the environment, and a similar work at the Bariri Reservoir were carried out. Eight sampling locations were established, four in each Reservoir. From these sampling locations, only one, BA1 (Barra Bonita – Point 1), was at the Piracicaba River, the main tributary of the Tietê River,. The sampling station BA2, was positioned upstream of this confluence. All other sampling locations were sequentially positioned downstream. Superficial water samples and sediments were sampled at each location, on March 2002, and April, July and October 2003. Water samples were analysed for total Hg and Se by AFS, using a PS Analytical Ltd instrument. Physico-chemical parameters (conductivity, turbidity, dissolved oxygen, temperature, total dissolved solids, pH and redox potential - Eh) were measured at the water column by using an analyzer HORIBA U-22. Sediment samples were analyzed for C, N, pH, Hg, Se and granulometry. For the estimation of potential toxicity of the sediments, analysis of acid-volatile sulfide (AVS) were performed and the concentrations of Pb, Cd, Zn, Ni and Cu (Simultaneous Extracted Metals – SEM) were determined by ICP-AES, using a Perkin-Elmer Optima 3000,. Results indicated very low concentrations of both Hg (< 2.00 ng.L-1) and Se (5.00 ng.L-1) in the water basins. Hg and Se were at ng.g-1 levels in sediments and probably the high concentrations of Hg previously observed in fishes catched at the Barra Bonita Reservoir, were either due to the methylation of the inorganic mercury in this compartment, or by an alloctone source. The ratio of AVS/SEM varied over a wide range of magnitude, depending on the season and on the sampling points, being Hg presented also in the AVS extracts (HCl 0.5 M). This could be an indication that this small fraction of the S2- reservoir could control de Hg availability, moreover due to the low activities observed for Se in these systems.

Key words: mercury, selenium, biogeochemistry, reservoirs, AVS

Introduction

Natural chemical cycles and movement of contaminants, occur faster in aquatic systems than in terrestrial ones, where through the food chain potentially reaches human consumers[1]. Rivers, which integrateing the land and the hydrosphere, and play a whole in the biogeochemical cycles of elements, are of great current concern [2]. Such is the case of the Tiete Hydrographic Basin, the biggest river of São Paulo State, which is 1000 km long, and receives a wide range of pollutants loads, including those from the agricultural activities, were considered potentially important source of Hg [4]. In addition to that, hydroelectric reservoirs can cause a rise of Hg content in edible fishes [5]. The Tietê River has over seven dams along its length, and in 1981, high concentration of Hg were detected in a carnivorous fishes (plagioscion squamossis), caught from the Barra Bonita Reservoir [6]. Selenium, an essential element, has been shown to reduce the bioaccumulation of Hg in aquatic systems[7], and has even been recommended as a possible treatment to reduce the contamination level of fish in Swedish Lakes[8]. The complexity of the global Hg cycle [9] is well recongized, and Se can be strongly involved [10]. The Barra Bonita and the Bariri Reservoirs are located about midway along the length of the,Tiete River. At these locations, we investigated Hg and Se biogeochemical interactions, in which Se controls the availability of Hg to the biota. Sampling of surface water and sediments were carried out in both ecosystems at different seasons, as described in the above abstract.

Results and discussion

In all situations the levels of Hg and Se in water were lower than the limit of detection (LOD) of the AFS method, being established in the experimental conditions as 2 and 5 ppt respectively. As observed in the water profiles, the pH varied slightly both with depth and spatially being alkaline. Eh decreases with depth, varying from negative to positive values. Although the concentrations of total Hg and Se were very low, with the observed range of Eh and pH variables, few chemical species of Hg as Se could be expected in these ecosystems. No thermal stratification was observed in the water column during the different sampling periods . The sediment granulometry, classified as total sand, silt and clay, varied between the two reservoirs and also from point to point in the same reservoir. Figure 1 indicates the degree of such variation for sediment samples collected at the Barra Bonita in July, 2003. The percentage of clay decreases from point BA1 towards BA4, positioned closer to the dam. It was observed smaller granulometry variations for two samples collected at same time for the same point. This is probably because the bottom sediment has been mobilized and re-suspended by sand exploration boats operating in both reservoirs. Although partitioning of metals to suspended sediment is known to be important, granulometry did not predict the Hg and Se concentrations in the sediment. Total Hg and Se

Figure 1 – granulometry percentage of particles presented in a sediment samples from the Bariri Reservoir, collected on July, 2003.  Positioned from left to the right, points BI1, BI2, BI3 and BI4 respectively, are separated in Km.

concentrations varied in the range of 0.04 to 0.6 ppm and 0.04 to 2.4 ppm respectively. By employing sequential approaches (acetic acid, hydroxylamine and hydrogen peroxide), only Hg and not Se were not detected in all extractants. On the other hand, very low concentration of Zn, Cu, Cd, Pb and Ni were observed. Volatile Acid Sulphate (AVS) and its ratio to the total amount of micro-equivalents of the above metals were also determined. Only Hg was detected in the acid extract, and at very low concentrations. The AVS/Zn,Cd,Ni,Pb,Cd varied widely, and was usually higher than 1, leading to a very low potentiality for metals delivery to the environment. Highest ratios were always observed at point BA2, the sampling location nearest to the pollution source on the Tiete River.

Conclusions

Environmental compartments, such as water and sediment, of the Barra Bonita and the Bariri Reservoirs do not present high concentrations of Hg and Se. Considering the observed values of the AVS/Zn,Cd,Ni,Pb,Cd, ratios, sediments shouldn’t be considered the source Hg and other metals to the biota, as shown out in 1981. The presence of low Hg contents and the absence of Se in AVS extracts indicates a competition between Se and S for Hg species in these ecosystems.

Acknowledgements

Our gratitude to Fundação de Amparo à Pesquisa do Estado de São Paulo – FAPESP, for the grant in the Research Project n0: 00/14460-3.

## References

[1] Burger, J & Snodgrass, J. (2001): Metal levels in Southern Leopard Frogs from the Savannah River Site: location and body compartment effects. Environmental Research Section.; Vol. 86, pp. 157-166.

[2] Wetzel, R.G. (1975): Limnology. W.B.Saundres. Philadelphia; 743 p.

[3] CETESB. Avaliação do níveis de contaminação por mercúrio na água, sedimento e peixes na Represa de Barra Bonita e seus rios formadores: Piracicaba e Tietê. São Paulo. CETESB. 1986. 115 p.

[4] Shanker, k., Mishra, S., Srivastava, S., Srivastava, R., Dass, S., Prakash, S & Srivastava, M.M. (1996): Study of mercury-selenium (Hg-Se) interactions and their impact on Hg uptake by the radish (Raphanaus sativus) plant. Food and Chemical  Toxicology.  Vol 34. pp. 883-886.

[5] Jackson, T. (1991): Biological and environmental control of mercury accumulation by fish in lakes and reservoirs of  Northern Manitoba, Canada. Canadian Journal of Fisheries and Aquatic Science, Vol. 48, No 12, pp. 2449-2470.

[6] Eysink, G.G.J.L., Pádua, H.B. & Martins, M.C. (1988): Presença do mercúrio no ambiente. Ambiente, Vol. 2, No. 1. pp. 43-50.

[7] Southworth, G.R., Peterson, M.J & Ryon, M.G. (2000): Long-term increased bioaccumulation of mercury in largemouth bass follows reduction of waterborne selenium. Chemosphere 41, pp.1101-1105.

[8] Paulsson, K & Lundbergh, K. (1991): Treatment of mercury contaminated fish by selenium addition. Water, Air and Soil Pollution 56, pp. 833-841.

[9] Mason, R.P., Morel, F.M.M & Hemond, H.F. (1995): The role of microorganisms in elemental mercury formation in natural waters. Water, Air and Soil Pollution 80, pp. 775-787

[10] Bjornberg, A., Hakanson, L & Lundbergh, K. (1988): A theory on the mechanisms regulating the bioavailability of mercury in natural waters. Environmental Pollution 49, pp.53-61.