On the Atmospheric Lifetime of Elemental Mercury

Ian M. Hedgecock, Giuseppe A. Trunfio and Nicola Pirrone

CNR - Istituto sull'Inquinamento Atmosferico, Rende, 87036, Italy, i.hedgecock@cs.iia.cnr.it

  - Abstract The lifetime of elemental mercury in the Marine Boundary Layer (MBL) has been studied as a function of time of year, latitude, temperature, liquid water content and cloud optical depth using the AMCOTS (Atmospheric Chemistry of Mercury Over the Sea) box model. The model includes gas and aqueous phase chemistry, and includes heterogeneous reactions as well as mass transfer processes. The sea salt and sulphate components of the ambient aerosol are treated separately. Photolysis rate constants are calculated as a function of latitude, time of year and cloud optical depth. Simulation of Hg0(g) depletion and HgII(g) production show that under clear sky conditions significant reduction (up to 90%) of Hg0(g) concentrations could occur within a week, in the absence of emissions, and that over the same period HgII(g) concentrations could reach more than 100 pg m-3.

Key words: mercury, modeling, lifetime, MBL

Introduction

The AMCOTS (Atmospheric Mercury Chemistry Over The Sea) model is a 0-dimensional combination of gas and atmospheric aqueous phase MBL chemistry, gas and aqueous phase Hg chemistry, (the aqueous phase being either sea salt or sulphate aerosols), mass transfer between phases and gas and aqueous phase photolysis. The chemical reaction database used to simulate the chemistry of the MBL is based on that used in the MOCCA model (Sander, 2001), the rate constants have been updated where appropriate with reference to von Glasow et al., (2002); the Hg chemistry included is described in  (Hedgecock et al., 2003, Hedgecock and Pirrone, 2004).

Simulations were performed starting with an initial Hg0(g) concentration of 4.5×106 molecules cm-3 (1.5 ng m-3 at 1 atmosphere, 298K). The other initial gas phase concentrations used were from Sander and Crutzen, (1997) and von Glasow et al., (2002). The aerosol number density and radii used, were those from MOCCA and the respective LWC values were 1.07×10-12 and 3×10-11 vol/vol for the sulphate and sea-salt particles respectively. Aerosol deposition is simulated using an atmospheric lifetime of 2 and 7 days for sea-salt and sulphate particles respectively. Dry deposition is modelled as a constant deposition velocity, for HgO, HgCl2, and HgBr2, 2 cms-1 was used, the same as that used in MOCCA for the inorganic acid gases HNO3, HCl and H2SO4. The air temperature was taken from the monthly averages for the Atlantic ocean (ISCCP, 2002). In all simulations a boundary layer height was assumed to be 1000m and clouds assumed to be above this if present. Cloud optical depths used were 10, 20, 40 and 60.

Results and Discussion

The reduction of the Hg0(g) concentration expressed as a percentage of the starting concentration is shown in Figures 1-5 for different latitudes (° N) in January and July for the five cloud optical depth

scenarios simulated.

| Figure 1 | Figure 2 |
| --- | --- |
| Figure 3 | Figure 4 |
| Figure 5 |  |

The influence of sunlight on Hg0(g) depletion is evident from both the generally lower depletion seen at 60° N in January, shorter days and the generally decreasing depletion seen with increasing cloud optical depth. There are some results in the simulations which at first sight seem a little odd. For example Hg0(g) depletion at 10° N in July has a maximum with a cloud optical depth of 20. The  Hg0(g) depletion at 30° N in January is higher than that at 10° N in the same month even though at 30° N the days are shorter. The reasons behind these apparent anomalies lie in the influence of air temperature, and the wavelength dependent attenuation of incoming solar radiation by clouds on the processes which lead to the production of reactive halogen species in the MBL, (Hedgecock and Pirrone, 2004). Cooler air temperatures increase the solubility of acidic gases in the sea salt aerosol increasing the rate of acidification and enhancing the rate of reactive halogen precursor species outgassing from the aerosol. Hence the greater depletion at 30° N where the air temperature is lower than at 10° N where it is warmer. One of the major sinks BrO (formed from O3 + Br, and rapidly photolysed to Br + O, and therefore in dynamic equilibrium with Br, recently identified as an important Hg0(g) oxidant (Ariya  et al., 2002), is HO2, which depends for its production on the  OH radical which is therefore strongly influenced by the rate of O3 photolysis. The attenuation of incoming solar radiation by clouds is greater at the shorter wavelengths which photolyse O3 than at the longer wavelengths which photolyse Br2 and BrCl, the two principal precursors for Br. Therefore, up to a certain point increasing cloud cover reduces HO2 production, limiting the BrO sink, whilst at the same time having a lesser effect on the photolysis of Br precursors. Thus under slightly cloudy conditions the Br concentration can be higher than under clear sky conditions enhancing Hg0(g) depletion. Using average cloud optical depth and surface air temperatures the lifetime of Hg0(g) can be calculated. The lifetime is the sum of the inverse of the oxidation reaction rates (rate constant multiplied by concentration):

 =  k(Hg0+O3).[O3] + k(Hg0+OH).[OH] + k(Hg0+Br).[Br] -1,

-1      2 × 10-8 + 8.7 × 10-8 + 1 × 10-6 thus,    10.5 days,

alternatively the lifetime with respect to the individual reactions under the model conditions are: (Hg0+O3) =  578 days,  (Hg0+OH) =  133 days and (Hg0+Br) = 11.5 days (Hedgecock and Pirrone, 2004).  The atmospheric lifetime of Hg0(g) therefore would appear to depend on which part of the atmosphere is being considered; in the free troposphere and in continental boundary layers the answer is probably around a year as is generally thought, although it should be borne in mind that the O3 concentration used in these studies (21 ppb) for the remote MBL is low compared to the concentrations found for example in the Mediterranean and also in some continental air masses. Higher O3 itself would not lead to a much lower lifetime, but the higher OH concentrations resulting from increased O(1D) would decrease the Hg0(g) lifetime with respect to OH oxidation. If the daily average concentration increased to say 5×106 molecules cm-3 the lifetime with respect to OH oxidation would decrease to a little less than 1 month. However global scale modelling studies have found that such a rapid rate of oxidation is difficult to reconcile with the stable hemispheric background concentrations, whilst noting that the reaction of Hg0(g) with O3(g) is too slow to account for all the Hg0(g) removal, (Bergan and Rodhe, 2001).  In the MBL, depending on time of year and atmospheric conditions, the atmospheric lifetime of Hg0(g) may well be as short as two weeks, and this would apply also to continental air masses moving over the sea. Bearing in mind that halogen activation is more rapid under polluted (high NOx) conditions (Sander and Crutzen, 1996) Hg0(g) from continental air masses could be deposited relatively close to the coast. This would particularly be the case in areas such as the Mediterranean.

Conclusion

Using a detailed multiphase photochemical box model of the MBL, it has been shown that it is  probable that Hg0 in the remote MBL can have an atmospheric lifetimes as short as 10 days, depending on time of year, cloud cover (optical depth) and latitude. The factors influencing the rate of Hg0 oxidation are the relative rates of O3 and Br2/BrCl photolysis, the air temperature and the length of the day. The role of Hg0 emission from the worlds oceans in global Hg budget estimations needs to be reassessed. This already been addressed to some extent by Mason and Sheu (2002), who have revised downwards their estimate of the contribution of the Hg0 evasion flux to the overall global budget, due to the more rapid cycling and re-deposition of Hg in the MBL when compared to the continental boundary layer. However it should now be possible to constrain these estimates even more closely, with a knowledge of average cloud optical depth, surface air temperatures and perhaps parametrizing aerosol production rates

.

## Acknowledgements

This work is part of the MERCYMS (http://www.cs.iia.cnr.it/MERCYMS/project.htm/  funded by the European Commission (Contr. No. EVK3-2002-00070).

References

Ariya, P.A., Khalizov, A., Gidas, A., (2002): Reaction of gaseous mercury with atomic and molecular halogens: kinetics, products studies, and atmospheric implications. J. Phys. Chem. A, Vol. 106, pp7310-7320,.

Bergan, T., Rodhe, H., (2001): Oxidation of elemental mercury in the atmosphere: Constraints imposed by global scale modelling. J. Atmos. Chem., Vol. 40 pp191-212.

Hedgecock, I.M., Pirrone, N., (2004): Chasing Quicksilver: Modelling the atmospheric lifetime of  in the marine boundary layer. Env. Sci. Technol., Vol. 38, pp69-76.

Hedgecock, I.M., Pirrone, N., Sprovieri, F., Pesenti E., (2003): Reactive gaseous mercury in the marine boundary layer: Modelling and experimental evidence of its formation in the Mediterranean region. Atmos. Environ., Vol. 37, S41-49.

ISCCP, (2002): International Satellite Cloud Climatology Project, Cloud analysis Part 3 Seasonal variations of cloud and surface properties. http://isccp.giss.nasa.gov/climanal3.html

Mason, R.P., Sheu, G.-R., (2002):  The role of the ocean in the global mercury cycle. Global Biogeochemical Cycles, Vol. 16 (4), p1093. doi:10.1029/2001GB001440

Sander, R., (2001): MOCCA and MoccaIce user manual for version 66, http://www.mcph-mainz.mpg.de/sander/mocca

Sander, R., Cruzten, P.J., (1996):  Model study indicating halogen activation and ozone destruction in polluted air masses transported to the sea. J. Geophys. Res., Vol. 101, 9121- 9138.

von Glasow, R., Sander, R., Bott, A., Crutzen, P.J., (2002): Modeling halogen chemistry in the marine boundary layer   1. Cloud free MBL, J. Geophys. Res., Vol. 107, 4341.