Carrying Capacity and Risk Assessments of Marine Ecosystem

Dr. Wilaiwan Utoomprurkporn1, Dr.Supichai Tangjaitrong1

Dr. Pornsook Chongprasith2

1Department of Marine Science, Chulalongkorn University, Phayathai Rd., Bangkok10330, Thailand, e-mail:wilaiwan@sc.chula.ac.th, supichai@sc.chula.ac.th

2Pollution Control Department, Ministry of Natural Resource and Environment, 92 Phahonyothin Rd., Phayathai, Bangkok 10400, Thailand,

e-mail:marinepollution_pcd@yahoo.com

Abstract : Carrying capacity and ecological risk in the coastal areas in Samut Prakarn Province and Map Ta Phut Industrial Estate in Rayong Province Thailand, where water quality is impaired by mercury from point and non-point sources, were studied. The assessment focused on mercury as the stressor and formulated Hazard Quotient (HQ) and Carrying Capacity Quotient (CQ) to reflect exposure levels of a particular type of organism to mercury.   RMA2 model was used to estimate hydrological states of the areas. Results of the hydrological model were then used by RMA4 model to determine fate of mercury in water column, which is controlled by the chemical, biological and hydrological interactions. Furthermore, an ecological study was performed to assess the components of the coastal ecosystems of the study areas especially on structures of the food chains and living organisms. The data obtained from the models and ecological study eventually was used to determine the stress condition values. For digestion rates of ingested food, which inherently reflect the acceptable or threshold levels of mercury concentration in environment, the values were extracted both from toxicological literatures and toxicology studies conducted in the study for local organisms.

The risk assessment showed that most of ecological components within the food chain of the coastal areas of Samut Prakarn Province are subjected to somewhat high risk. Only organisms within the same level of the food chain as the keystone species (benthos) are not yet under such high risks while primary producers have lower risks. At Map Ta Phut Industrial Estate, the average risk levels were relatively lower compared to Samut Prakarn Province. The components of the food chain, from primary producers to top consumers, are under low risk conditions. In both study areas, it was observed that shrimps, which are bottom feeders, are under relatively high risks. The carrying capacity assessment revealed that most areas of the both study sites still have capacity of dilution effect. However, in few areas where some organisms are considerably sensitive to mercury, carrying capacity was very low.It can be concluded that the coastal areas of Samut Prakarn Province and Map Ta Phut Industrial Estate in Rayong Province are not severely impaired by mercury released from several sources. However, some living organisms are facing high risk levels.

Key words: carrying capacity, risk assessment, hazard quotient, carrying capacity quotient

# Introduction

Carrying capacity assessment is the evaluation of the coastal ocean in sustaining a certain amount of substances, or pollutants, before they reach levels that generate toxic conditions, which are harmful to the particular ecosystem.  In other words, it is the assessment of a coastal area’s ability to dilute and maintain certain concentrations of substances, or pollutants, at levels, which pose no threats to the stability of the area’s ecosystem.

Ecological risk assessment is the process for defining the probability of the degradation of an ecosystem induced by certain impending threats.  In fact, such threats maybe intense and short-termed, or moderate, but remain for a long period of time, which ultimately result in both temporary and permanent detrimental effects.

Both the ecological carrying capacity and risk assessments require basic information concerning the composition of the biological community within the ecosystem.  Particularly, the transfer pollutants with in the biological food web, which can be divided into two types: the grazing food web (starts from the primary producers) and the detritus food web (starts from nonliving organic substances).

Generally, physical and chemical processes result in the accumulation of contaminants in the sediments on the seafloor.  Although the concentration of pollutants in the sediment layers is low, long-tern accumulation may affect the physiology and survival of the organisms residing within the ecosystem.  Furthermore, high concentration of pollutants may cause immediate toxicity resulting in the fatality of affected organisms.

It is for these reasons that the ecological carrying capacity and risk assessments (Figure1) are focused on the effects of both low and high concentration pollutants on diversity and abundance of living organisms in the ecosystem.  In particular, the change in the composition of the ecosystem caused by the effects of pollutants to richness and productivity of the target and surrounding areas.  Furthermore, it is crucial that the effects on humans caused by consumption of contaminated marine products and water are seriously considered.

Determination of Ecological Effects

Contaminated Ecosystem

Ecological Study

Toxicology Study

Chemical Oceanography Study

Physical Oceanography Study

Dispersal of Pollutant

Distribution of Living Organisms in the Ecosystem

Determination of Toxic Concentration

Pattern of Dispersal

Establish a relationship among pollutants

Assessment

Highest concentration which does not produce negative effects

Assessment

Carrying Capacity

Risk

Figure 1  Flow chart of procedures for risk and carrying capacity assessments.

## Results and discussion

In the study, the results of ecological risk and carrying capacity assessment were shown in form of Hazard Quotient (HQ) and Carrying Capacity Quotient (CQ) respectively. The HQ and CQ value can be derived from the following formulas:

HQ = EEL/TRV 	and	     CQ = (TRV-EEL)/TRV

where EEL = stressed conditions and TRV = digestion rate of ingested food.

It is apparent that the determination of HQ and CQ value relies on both EEL and TRV. EEL value by definition reflects the exposure level of a particular type of organism to mercury. Obviously, the magnitude of EEL is dependent on concentration of mercury in water column and exposure characteristics (direct contact, food chain or both paths). The determination of mercury concentrations in water column is complicated because mercury concentrations are considerably dependent on time and space. Normally, the fate of mercury in water column will be governed by the chemical, biological and hydrological interactions in water column and sediment. Moreover, time affects the mercury level in water column mostly because of tidal activity taking place within water column. In order to facilitate the calculation process of mercury concentrations, the mathematical models called RMA2 and RMA4 were introduced as accessory tools for conducting calculations under a specific boundary condition and displaying the results in 2-dimension graphic format. Furthermore, the ecological study was performed to assess the components of the coastal ecosystems of Samut Prakarn Province and Map Ta Phut especially the structures of the food chains and living organisms. The data obtained from the models and ecological study eventually was used to determine EEL value. Evidently, high EEL reflects the high exposure level to mercury. The toxicology study was also conducted to provide the database for the calculation of TRV value. In fact, TRV value inherently reflect the acceptable levels (or threshold level) of mercury concentration in environment that have no toxic effects on living organisms residing in the particular environment

With the results from the models, ecological study and toxicology study, it is possible to calculate EEL and TRV values, which subsequently were used to determine Hazard Quotient (HQ) and Carrying Capacity Quotient (CQ). The magnitude of HQ and CQ value is of great importance in the assessment of carrying capacity and ecological risk in both coastal areas. If HQ >> 1, it means that the risk that mercury level in tissues of a living organism exceeds an acceptable level (based upon toxicology study) is substantially high. If HQ << 1, its meaning will be contrary. It is conspicuous that high CQ indicates that the location still has high capability of diluting mercury to the level that pose no threat to the stability of the ecological system. It should be strongly aware that HQ and CQ are variable with space, time and type of organism.

From the risk assessment, it was found that most of ecological components within the food chain of the coastal areas of Samut Prakarn Province are subjected to somewhat high risk. Only organisms within the same level of the food chain as the keystone species (benthos) are not yet under such high risks while primary producers have lower risks. At Map Ta Phut Industrial Estate, the average risk levels were relatively lower compared to Samut Prakarn Province.  The components of the food chain, from primary producers to top consumers, are under low risk conditions.  In both study areas, it was observed that shrimps, which are bottom feeders, are under relatively high risks. The carrying capacity assessment revealed that most areas of the both study sites still have high capacity of dilution effect. However, in few areas where some organisms are considerably sensitive to mercury (referred to toxicology study), carrying capacity (very low CQ) was very low.

Conclusion

It is obvious that carrying capacity and ecological risk assessment are substantially beneficial to evaluate the extent of threat caused by mercury and the ability of the study area to degrade mercury into the level that is not harmful to a particular organism. It is concluded that the coastal areas of Samut Prakarn Province and the coastal areas of the Map Ta Phut Industrial Estate in Rayong Province overall are not severely impaired by mercury released from several sources. Even if overall conditions are acceptable, some living organisms are facing high risk level and will certainly affect human health. Hence, optimization is needed to find to best way (best management practices) to minimize the risk level in some organisms by taking advantage of database obtained from the carrying capacity and ecological risk assessment.

### Acknowledgements

This work was supported by the Pollution Control Department, Ministry of Natural Resources and Environment, and carried out by Faculty of Science, Department of Marine Science, Chulalongkorn University, Bangkok, Thailand.

## References

Garrels, R. M. and Mackenzie, F.T. 1971. Evolution of Sedimentary Rocks. NewYork: W.W. Norton, 397 pp.

Snidvongs, A. 1993. Geochemistry of organic particulates in shallow water continental shelf environments. Ph.D. Dissertation, University of Hawaii, 201 pp.Budavari, S., M.J. O’Neil, A. Smith, and P.E. Heckelman. 1989. The Merck Index: An Encyclopedia of Chemicals, Drugs, and Biologicals. 11th Edition. Merck and Company, Inc. Rahway, New Jersey

Suter, G.W. II. 1993. Ecological Risk Assessment. Lewis Publishers. Chelsea, Michigan. 538 p.

U.S. EPA. 1999.  Screening Level Ecological Risk Assessment Protocol for Hazardous Waste Combustion Facilities. EPA 530-D-99-001A, August.

.