Contamination of mercury in Buyat Bay, North Sulawesi, Indonesia:

bioaccumulation, contamination status, and potential impact to human

Markus T Lasut1 & Yoshiaki Yasuda2

1Faculty of Fisheries and Marine Science, Sam Ratulangi University, Jl. Kampus Unsrat Bahu, Manado 95115, Sulawesi Utara, Indonesia, E-mail: M.Lasut@unsrat.ac.id
2National Institute for Minamata Disease, 4058-18 Hama, Minamata City, Kumamoto 867-0008, Japan, E-mail: y2the2nd@nimd.go.jp

Abstract: The marine environment of Buyat Bay, North Sulawesi, Indonesia, is suffered contamination by mercury discharged from submarine tailings of a gold mining industry. The concentrations of mercury in marine sediment and biota (fish, bivalve and gastropod, seagrass and macro-algae, soft coral) as well as human’s hair were examined in the present study. Bioaccumulation in trophic level of marine environment, contamination status, and potential impact to human are characterized and discussed in this paper.

Key words: mercury, bioaccumulation, biomagnification, submarine tailings disposal (STD), Minahasa, Sulawesi, Indonesia.

Introduction

Figure 1. Location of study at Buyat Bay, North Sulawesi Province, Indonesia.

Buyat Bay

Manado City

Since 1996, a submarine tailings disposal (STD) system has been applied by a gold mining company to discharge 2000 tons per day of detoxified mercury-containing tailings (45-55% solid of <75 µm) of cinnabar ores through a pipeline to ±82 m depth (±900 m horizontally) of marine area of Buyat Bay, North Sulawesi Province, Indonesia (Fig. 1). As various minerals-associated pollutants, including mercury, can be found in the tailings[1], the coastal and marine environment of the bay are predicted being contaminated due to the application[2], besides from another land-based source of mercury-using activities (for instance: artisanal gold mining). The objective of this study is to evaluate the contamination and the source of contamination, as well as discussing bioaccumulation-biomagnification processes, contamination status, and potential impact of mercury to human.

Sampling has been conducted in early February 2004 where several sets of samples from coastal and estuary areas of the bay have been measured for total mercury (T-Hg). The

Right                                               Left

St#1

St#2

St#3

St#4

St#5

St#6

STD pipeline

Outfall

River of Sungai Buyat

Figure 2. Sampling sites (St)

Buyat Bay

samples of biota were selected according to marine trophic level, such as seagrass, Enhalus acoroides, (n=10); seaweed, Turbinaria sp., (n=10); soft coral (n=10); bivalve, Septifer sp., (n=10) and oyster (n=3); gastropod, Nerita sp., (n=10); coastal crabs (n=10); and fish, Priacanthus hamrur, (n=10), and Horse Mackerel (n=10). Sediment was sampled at 5 sites (St#1-5) along the beach (St#4 is at the river mouth of Sungai Buyat) and one site (St#6) at river estuary (±250 m inward from the river mouth) (Fig. 2); 5 replicates were done in each site. These sampling sites were designed in order to investigate the source of contamination, whether from the tailings or the land-based source. Human’s hairs, male (n=10) and female (n=15), were measured for T-Hg and organic mercury (Me-Hg). All mercury measurements[3] were performed in the laboratory of Natural Science Laboratory, Minamata Disease Archive, National Institute for Minamata Disease, Japan.

Table 1. Average concentration of T-Hg (mg/kg wet weight) in different sampling sites (average ± STD).

|  | St#1 |  | St#2 |  |  | St#3 | St#4 |  |  | St#5 |  |  | Average |  | St#6* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sediment** | 0.060 ± 0.040 |  | 0.057 ± 0.022 |  |  | 0.071 ± 0.061 | 0.041 ± 0.007 |  |  | 0.063 ± 0.011 |  |  | 0.064 ± 0.036 |  | 0.093 ± 0.077 |
|  | middle side (St#2-3) |  |  |  | right side (St#4-5) |  |  |  | Average |  |  |  | river estuary (St#6) |  |  |
| Crabs | 0.035 ± 0.006 |  |  |  | 0.039 ± 0.013 |  |  |  | 0.037 ± 0.010 |  |  |  | 0.084 ± 0.018 |  |  |
|  |  |  |  | along the beach |  |  |  |  |  |  |  |  |  |  |  |
| Bivalve (Septifer sp.) |  |  |  | 0.056 ± 0.010 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | St#1 |  |  |  |  |  |  |  |  |  |
| Seaweed (Turbinaria sp.) |  |  |  |  |  | 0.014 ± 0.003 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | St#1 |  |  |  |  |  |  |  |  |  |
| Soft coral |  |  |  |  |  | 1.312 ± 0.0557*** |  |  |  |  |  |  |  |  |  |
|  |  | left side (St#1-3) |  |  |  |  |  | right side (St#4-5) |  |  |  |  |  | average |  |
| Gastropod (Nerita sp.) |  | 0.123 ± 0.031 |  |  |  |  |  | 0.044 ± 0.005 |  |  |  |  |  | 0.083 ± 0.018 |  |

St: site; *river estuary; **dry weight, ***µg/kg wet weight

Results and discussion

The present study suggested that the environment of Buyat Bay is contaminated by mercury in various levels. This was indicated by the results that mercury (T-Hg) was found in all biota samples (Table 1 and 2) as well as the sediment. The concentration varied from the lowest in soft coral to the highest in fish. This variation was perhaps due to each species of biota has its own strategy to accumulate the mercury.

With regard to the marine trophic level, the concentration of T-Hg tended to increase in the biota as their positions in the trophic level increased (Fig. 3). This can be seen from the result that the relative low level was in the producer’s organisms (soft coral, seagrass, and seaweed), while the relative high was in the high consumer of the fish. This may indicate that bioaccumulation-biomagnification processes of mercury occurred in this area. However, it was not the case in the coastal crabs (a carnivorous species), which have low concentration; this was due in part to the shell that reduces concentration value of sample.  It is hard, however, to remove shell from the crab body, which may account for around 50% of whole body weight.

Figure 3. Average T-Hg concentration in the marine biota of Buyat Bay [a: seagrass (E. acoroides), whole part; b: seaweed (Turbinaria sp.); c: bivalve (oyster), overcoat film part; d: bivalve (Septifer sp.), soft body part; e: gastropod (Nerita sp.), soft body part; f: marine crabs, body part; g: fish (horse mackerel), meat part; fish (P. hamrur), meat part].

The T-Hg concentration level in sediment was different in each sampling site that shows an unstable gradient along the beach line from St#1 to #5. Within the beach sediment, the comparison was St#3>St#5>St#1>St#2>St#4. The level in sediment at the river estuary (St#6) was different then those in the beach (Table 1). However, it was statistically not significant (p>0.05). The difference shown above may be attributed to the different source of mercury input, that is, the relative high concentrations of T-Hg at St#3 and St#1 were maybe due to the submarine tailings; while those of St#5 and St#6 were from the land-based source. However, more investigations are needed to confirm this result. Distribution of the submarine tailings to the coastal area of the bay was previously observed in the 18-20 m depth and in the 1 km distance from the tailing outfall[4].

Table 2. Average concentration of T-Hg (mg/kg wet weight) in several organs of marine biota in Buyat Bay (average ± STD).

|  | OCF | AM | DG |
| --- | --- | --- | --- |
| Bivalve (oyster) | 0.014 ± 0.008 | 0.024 ± 0.023 | 0.107 ± 0.028 |
|  | Meat | Gill | Liver |
| # Fish # -P. hamrur | 0.199 ± 0.072 | 0.057 ± 0.018 | 0.868 ± 0.438 |
| -'Horse Mackerel' | 0.170 ± 0.053 | 0.040 ± 0.007 | 0.143 ± 0.028 |
|  | Root | Trunk | Leaf |
| Seagrass (E. acoroides) | 0.007 ± 0.001 | 0.005 ± 0.002 | 0.005 ± 0.001 |

OCF: over-coat film; AM: adductor muscle;

DG: digestive gland

The highest concentration of T-Hg in the fish meat (0.199 and 0.170 mg/kg wet weight) was lower than the WHO’s international human consumption advisory limit of fish (0.5 mg/kg wet weight). However, potential impact to human may occur as the amount of mercury (T-Hg and Me-Hg) in the human’s hair.

Mercury accumulated (T-Hg and Me-Hg) in human is shown in Fig. 4 based on human’s hair mercury measurement of those resident very close to the area. Apparently, male (4.19 and 4.16 mg/kg) is suffering from mercury contamination more than female (2.13 and 2.20 mg/kg). However, no control area is investigated in this study, and then all the comprehensive evaluation of mercury distribution should be done after the control study.

Figure 4. T-Hg concentration in hairs

Conclusions

Our results suggest that mercury is accumulating in the marine environment (sediment and biota) of the bay in various levels, including residents.

Acknowledgements

The present work performed under the collaborative study between Faculty of Fisheries and Marine Science, Sam Ratulangi University, and National Institute for Minamata Disease, Japan, entitled ‘impacts of submarine tailings disposal of gold mining activities on marine organisms and human health in north Sulawesi, Indonesia’.

References

[1] Ginting, A.R. (1999): Perkimiaan pada ekstraksi emas dan detoksifikasi limbah (Eng: chemistry in gold extraction and wastewater detoxification. In: Ginting, A.R (ed) Proceeding: Submarine Tailing Placement (STP), Seminar 2 Hari (15-16 Juli 1999), pp 24-37.

[2] Lasut, M.T. (2001): Submarine tailings disposal (STD) of gold mining activities: impacts on marine organism and human health. Proceeding of International Conference on Submarine Tailings Disposal, 23-30 April 2001, Manado, North Sulawesi, Indonesia, 10 p.

[3] Akagi, H., Nishimura, H. (1991): Speciation of mercury in the environment. In: Suzuki, T., Imura, N., Clarkson, T.W. (eds) Advances in mercury toxicology. Plenum Press, New York, pp 53-76.

[4] Anonymous (1998): RKL/RPL Report period October 1-December 31. PT. Newmont Minahasa Raya. 34 p.