Human Exposure to Mercury in the Riverine Population of the Madeira River, Amazon, Brazil

Ronaldo Cavalcante de Oliveira1; Wanderley Rodrigues Bastos1; Ene Glória da Silveira1 & Marlon de Freiras Fonseca2.

1Environmental  Biogeochemistry Laboratory, Federal University of Rondônia. BR-364, km 9,5 - Porto Velho/RO, Brazil. labmerc@unir.br.

2Radiosotopos Laboratory Eduardo Penna Franca, Biophysical Institute

Carlos Chagas Filho, Federal University of the Rio de Janeiro – UFRJ, CCS,

Ilha do Fundão, 21949-900, RJ, Brazil. marlon@biof.ufrj.br

Abstract: The objective of this work was to evaluate the total Hg contamination in riverine populations of the Madeira River, where intense gold mining activity occurred, mainly in the 70’s and 80’s. This population is characterized especially by high daily ingestion of fish, sometimes eaten twice a day. For this evaluation, human hair samples were collected and dietary questionnaires obtained from 217 residents of 13 sites in the year 2001. The total Hg concentration was determined by atomic absorption spectrophotometry coupled with cold vapour generation (FIMS-400 Perkin Elmer). Values between 0.63 g.g-1 and 63.54 g.g-1 were obtained in the hair samples. The average Hg values of the sampling sites varied from 6.30 g.g-1 to 36.89 g.g-1. In questionnaires, it was verified that 80.1% of the appraised populations consume fish daily. Relating the values of Hg with the weekly frequency of fish consumption, a significant correlation (r2=0.9184) was obtained. 86.8% of the appraised individuals showed hair Hg concentration over the World Health Organization recommendation of 6.00g.g-1.

Key words: mercury, human hair, riverine population, Amazon.

## Introduction

Mercury (Hg) and its compounds are considered persistent pollutants in the environment and highly toxic. It could originate from both natural and anthropogenic sources (Ferreira & Appel, 1990). In the Amazonian region the main anthropogenic source of Hg contamination in the last decades has been its use in gold extraction in the amalgamation process. Besides these mining activities, a great regional deforestation has occurred which contributes to the release of natural Hg from the soil (Lacerda & Salomons, 1992).

In the aquatic system, processes of transformation of inorganic Hg to methyl-Hg can occur (Pfeiffer et al, 1990). The largest concern regarding environmental mercurial pollution is about this organic form and its effects on human health.  The methyl-Hg can be found in aquatic organisms, mainly in carnivorous fish. The riverine population of the Madeira River has fish as its main protein source. In recent years, some studies have demonstrated that local fish contain Hg concentrations above that advisable for human consumption, putting that population at risk from environmental Hg contamination. Actually, the majority of people that live there have fish daily and, sometimes, twice a day. Thus, they are considered highly exposed. In this study, human hair samples of riverine populations were collected around the Madeira River in the Rondônia and Amazon states of Brazil (Figure 1).

Questionnaires with an alimentary focus were applied to volunteers in order to obtain data about fish ingestion. The human hair samples were collected close to the scalp of the occipital area by the use of stainless steel scissors, in locks of approximately 500 mg, soon afterwards transferred to plastic bags and kept at to room temperature.

In the laboratory, the samples were washed with 20,0mL of 0.01% EDTA. Afterwards they were cut up with scissors for better homogenization of the selected sample. The total Hg concentration was determined by the technique of atomic absorption spectrophotometry coupled to a cold vapour generation FIMS–400 (Flow Injection Mercury System - Perkin Elmer) (Bastos et al, 1998).

Figure 1. Map of the study area.

Results and Discussion

All of the appraised communities in this study had average total Hg concentrations above the biological limit of tolerance of 6.00 g.g-1 established by the World Health Organization (WHO). It was also confirmed, through the alimentary inquiry, that 80.1% of the residents of the studied areas consume fish daily and on average at two meals a day (Figure 2).

Table 1. Total average Hg content in human hair of the riverine population of the Madeira River.

| Sampling sites | Hg (g.g-1) | S.D. | Min | Max | n |
| --- | --- | --- | --- | --- | --- |
| Riverside | 17.75 | 7.65 | 6.30 | 36.89 | 217 |
| WHO limit | 6.00 | - | - | - | - |

Eightyseven percent of the riverine individuals studied contained Hg concentrations above the limit suggested by WHO. The results of the present study corroborate other research accomplished in the Amazonian riverine communities, suggesting that these populations constitute an important group at risk from Hg contamination due to the high fish ingestion. These data suggest that these people are exposed to high doses of methyl-Hg in this region.

1 - 3

4 - 5

6 - 7

Figure 2. Weekly frequency of fish consumption for the riverine population of the study area.

Relating the values of Hg in hair with the weekly frequency of fish consumption, a reasonable correlation was obtained:  r2 =0.9184 (Figure 3).

1 - 3

4 - 5

6 - 7

Figure 3. Total Hg concentration in relation to weekly frequency of fish consumption.

Conclusion

In this region, Hg burden of the riverine population is directly related to the consumption of local fish. The ingestion of fish constitutes a potential risk for the emergence of symptoms and signs of Hg intoxication, and environmental monitoring associated with prevention measures and control is indispensable. Total Hg levels are being determined in each local species of fish. Clinical studies are being started in Madeira river basin by the UNIR/UFRJ team.

Acknowledgements

This work was supported by the UNIR, PNOPG/CNPq No. 55.0882/01-4, RIOMAR Foundation and SEBRAE-RO. Our thanks are due to the Rio de Janeiro State Government for support that resulted in a specialist doctor in toxicology (PMERJ).

## References

Bastos, W.R.; Malm, O.; Pfeiffer, W.C. & Cleary, D. (1998). Establishment and analytical quality control of laboratories for Hg determination in biological and geological samples in the Amazon-Brasil. Ciência e Cultura Journal of the Brazilian Association for the Advancement of Science, V.50(4), 255-260.

Ferreira, R.C.H. & Appel, L.E. (1990). Fontes e usos de mercúrio no Brasil. In: Proceedings of Forest’90. Rio de Janeiro (contribuição técnica).

Lacerda, L.D. & Salomons, W. (1991). Mercury in the Amazon: a chemical time bomb. Dutch Ministry of Housing, Physical Planing and Environment, Haren.

Pfeiffer, W. C & Lacerda, L. D. (1988). Mercury imputs to the Amazon region, Brazil. Environ. Technol. Lett. London, 9-p. 325-350.

Pfeiffer, W.C.; Malm, O.; Souza, C.M.M.; Lacerda, L.D.; Silveira,E.G. & Bastos, W.R. (1990). Mercury in the Madeira river ecosystem, Rondônia, Brazil. Forest Ecology and Management. Elsiever Science publishers B.V. Amsterdam - Printed in the Netherlands.