Mercury in the Madeira River Basin, Rondônia, Brazil

## João Paulo O. Gomes; Deborah Pereira Linhares; Elisabete Lourdes Nascimento; Ailton Rodrigues Ferreira; Ândrio dos Santos Ribeiro; Éderson Rodiney Dantas Rodrigues; Wanderley Rodrigues Bastos & Ene Glória da Silveira.

Laboratório de Biogeoquímica Ambiental, Universidade Federal de Rondônia - UNIR. BR 364, Km 9,5 – +55 69 217-8538 – Porto Velho – RO, Brazil. labmerc@unir.br.

Abstract:

With objectives of evaluating the environmental impact caused in the basin drainage of the Gold mining activity reached a peak intensity in the Madeira River watershed during the1970’s and 1980’s. In order to assess the environmental impact, we assessed Hg levels in two abiotic matrices: bottom sediments and suspended solids in the Lower Madeira River. Chemical extraction of Hg adsorbed to particles was determined following the Bastos et al. (1998) methodology and quantification of Hg was determined by cold vapor generation atomic absorption spectrophotometry. It was observed that generally, Hg concentrations in the lakes, both in bottom sediments (66.26  34.80g.kg-1) and particles in suspension (34.25  28.71g.kg-1), were higher than concentrations found for Madeira River in the marginal bottom sediments (57.49  22.55g.kg-1) and in the suspended particles (27.38  5.97 g.kg-1). However, all of the concentrations are below the value of the world average values established by WHO (200 g.kg-1).

Key words: Amazon, mercury, bottom sediment, suspended solids.

## Introduction

The Madeira River is a tributary of the southern margin of the Amazon River. It has 1,459 km of reach and average flow of 23,000 m3/s. It is formed by the confluence of the Mamoré and Beni rivers, draining part of the Andean plateau. It is divided into two different sections along its course. The upper section, called Upper Madeira, extends between the origins up to Porto Velho. It is mostly non-navigable due to several rapids. The lower section, Lower Madeira, is totally navigable for a reach of 1,099 km between Porto Velho-RO and Itacoatiara-AM. According to Sioli et al. (1967), it is classified as a white-yellowish waters river, rich in dissolved organic matter and suspened solids.

In Rondônia State there was a high anthropogenic activity between the 1970’s and 1980’s with the large elemental mercury (Hg0) releases  from t gold mining amalgamation. The loss of Hg in that process accounted for up to 40 to 45% of the total Hg used, as effluent was discharged directly in the river and with the remaining of Hg evaded to the atmosphere (Pfeiffer & Lacerda, 1988). Ferreira & Appel (1990) estimate about 370 tons of Hg were introduced in the region just for the year 1989. About 160 tons were not recovered indicating that it was most likely transferred to the regional environment. There are no present official records for the Hg releases into atmosphere and aquatic environment in the region, and gold mining continues at a lower activity level compared to previous decades.

The Hg released directly in the aquatic systems tends to be adsorbed onto suspended solids and transported, which in time influence the deposition characteristics of lakes and tributaries (“igarapés”) during the flood and dry seasons, making these systems reservoirs of Hg sedimentation. The fine sediments are a useful proxy for the environmental Hg concentration and provide information about past deposition. Several other factors influence the geochemical behavior of Hg, such as the pH, redox potential and solids interaction and influence partitioning between the dissolved and particulate phase. It is a key process in the transport and possible bioaccumulation of Hg in the environment (Bonotto & Silveira, 2003).

Hg volatilizes and dispersed through atmosphere, and could have a long residence in the atmosphere during dry periods. Elemental Hg could also undergo chemical transformation in the atmosphere (oxidation) and be deposited locally through rainfall.

The study area is illustrated in  Figure 1> A total of 28 samples were collected along the course of Madeira: 8 sedimentation zones of Madeira River, 18 points in the lakes and 2 points in the “igarapés” that receive influence of Madeira River during the flood events. Bottom sediments samples were collected with an Eckmam dredge", sieved through a 200 um mesh. Suspended solids samples were collected on cellulose membranes (0.45m pore size). Samples were extracted withHCl-HNO3 (3:1) and oxidized with 5% KMnO4 . During analysis samples were reduced with NH2OH.HCl and NaBH4. Certified reference samples (IAEA-356) were run, with 97% recovery. Total Hg was quantified by cold vapor generation coupled atomic absorption spectrophotometer (Bastos et al, 1998).

Itacoatiara-AM

Porto Velho - RO

Figure 1. Location of the study area: Low Madeira River.

Localization of the study area

Results and Discussion

Hg concentrations in bottoms sediments in the sedimentation areas of Madeira river varied among 32.53 to 82.18g.kg-1 (Figure 2)  In lakes, Hg varied from 8.16 to 148.15g.kg-1 and in the “igarapés” from 38.8 to 48.16g.kg-1.  The highest Hg concentrations were found in the bottom sediments of lakes, due their hydrodynamic characteristics as reservoirs, with sedimentary deposition and accumulation of Hg.  In general, the Hg levels in bottom sediments of lakes were greater than those of the Madeira River and "igarapés".  However, the lakes presented a high variability of Hg concentration in drainage basin that possibly is related with the hydrodynamic micro-basin and are influenced by antropic and natural Hg dispersion. However, the Hg concentrations found were below the world average.

Max.

[Hg] g.kg-1

Std. Dev.

Mean

Std. Dev.

Min.

1-6: Madeira River              7-16: Lakes              17-18: Igarapés

Figure 2.  Hg concentration in bottom sediments

Levels of Hg in suspended solids (Figure 3),  varied from 20.00 to 33.30 g.kg-1 in the Madeira River and from 10.00 to 84.58 g.kg-1 in lakes. The Hg levels in the Madeira River showed a smaller variation in relation to lakes (27.38 + 5.97 g.kg-1). Large differences in the average Hg concentration along of particles in the Madeira basin, reflect hydrodynamic differences of flow and suspended solids between the main channel and the lakes. Seasonal influences can resuspend Hg-rich particles and remobilized those particles. This is of great concern because Hg associated with river sediments may be constantly remobilized and redeposited, making the Hg continually   available for uptake and bioaccumulation through the food chain.

Max.

[Hg] g.kg-1

Std. Dev.

Mean

Min.

Std. Dev.

1-5: Madeira River                                6-16: Lakes

Figure 3.  Hg concentration in suspended solids.

Conclusions

These results show that Hg is present in the aquatic system of Madeira River basin and dispersed through  resuspension and redeposition of suspended solids. This allows Hg to be transported to lakes and "igarapés" a great distance from gold mining activities in the region. This environment may also influenced be by erosion of soils, agriculture, and burning of the forests that additionally affect Hg levels in waters of the Andean drainage basin.

Acknowledgements

This work was supported by the UNIR, PNOPG/CNPq No. 55.0882/01-4, RIOMAR Foundation and SEBRAE-RO.

References

Bastos, W.R; Malm, O; Pfeiffer, W.C. & Cleary, D. (1998). Establishment and analytical quality control of laboratories for Hg determination in biological and geological samples in the Amazon-Brasil. Ciência e Cultura Journal of the Brazilian Association for the Advancement of Science, V.50(4), 255-260.

Bonotto, M.D. & Silveira, E.G. (2003). Preference rations for mercury and other chemical elements in the Madeira River, Brazil. Journal of South America Earth Science 15 911-923.

Ferreira, R.C.H. & Appel, L.E. (1990). Fontes e usos de mercúrio no Brasil. In: Proceedings of Forest’90, Rio de Janeiro (contribuição técnica).

Pfeiffer, W. C & Lacerda, L. D. (1988). Mercury imputs to the Amazon region, Brazil. Environ. Technol. Lett., London, 325-350.

Sioli, H. (1967). Hydrochemistry and geology in the Brasilian Amazon, Amazon region, Kiel, 1(3): 267-277.