Atmospheric Mercury Dispersion in Porto Velho City - RO, Amazon.

Grazyelle Sebrenski da Silva; Wanderley Rodrigues Bastos & Ene Glória da Silveira.

Environmental Biogeochemistry Laboratory, Federal University of Rondônia. BR-364, Km 9,5 - (69) 217-8538 - Porto Velho/RO, Brazil. labmerc@unir.br.

Abstract: Although there was a significant reduction in gold mining activity in the Amazon in the recent years, Porto Velho, capital of Rondônia state, still has a total of five gold workshops in operation. It was possible, through this research, to evaluate the mercury concentration in superficial atmospheric sediment from streets (that is, dust), in which the average interval between the minimum and maximum values in the year 2003 was 109– 1932 µg.kg-1. Besides the environmental study, an occupational evaluation of the gold shop workers was carried out, as well as an evaluation of atmospheric air from inside the workshops. The mercury concentration in the atmospheric air inside the workshops did not exceed 0.0050µg/m3. Concerning the occupational evaluation, the urine mercury concentrations were between 0.90 – 41.57µg.L-1, thus not exceeding the occupational limit of 50.00µg.L-1 (WHO).

Key word: Amazon, mercury, atmospheric, occupational

# Introduction

Elementary mercury vapour is possibly oxidized in the atmosphere through catalytic reaction with O3 and solar energy. The ionic species Hg+2 is removed from the atmosphere by pluvial waters and precipitated in aquatic and terrestrial environments where it is potentially reactive, and may be transformed to organic species which are capable of being assimilated by biota (Lindqvist et al, 1984).

In the Amazon, many study results indicate the presence of mercury in rivers, forests, in fish and consequently in the human population. In atmospheric air, high concentrations of mercury were found near smelting operations in gold mining activity and in gold workshops in Amazonian cities (Pfeiffer et al, 1992).

In Rondônia, gold mining activity started in 1978, increasing until 1985, when it started to reduce (Malm, 1991).

Nowadays, with the reduction of gold minig activity, the number of gold workshops in Porto Velho has also diminished (Figure 1). Those gold shops are the main source of mercury vapour in the region, for they perform the final ignition of the gold-mercury mixture.

Through the monitoring that has been carried out in the Amazon region it is possible to map the dispersion of metallic mercury vapour in the commercial area of Porto Velho.

Figure 1. Area of study in Porto Velho.

## Material and Methods

The collection of superficial atmospheric sediment in the streets (that is, dust) was made in a one year period at 10 sampling points near gold workshops in Porto Velho, between the dry and wet period. The samples were stored in plastic bags and subjected to a sieving process for particle selection (<200 mesh) in the laboratory.

The atmospheric air samples were collected by a bubbler system with controlled flowrate containing an oxidant solution (KMnO4 + H2SO4) (Figure 2). This system was operated inside the gold workshops for two hours.

In the occupational evaluation, 20 urine samples from 10 gold workshop workers were collected at the beginning and end of each day’s travel to work, by the distribution of polyethylene bottles to each worker. At the same time, informed consent of the participants was given.

For the determination of mercury concentration, the samples underwent chemical digestion with subsequent analysis by atomic absorption spectrophotometry coupled to cold vapour generation (FIMS-400 da Perkin Elmer). The analytical methodology used here was based on the techniques developed by Bastos et al (1998).

Figure 2. System of air collection inside the gold workshops.

Results and Discussion

The sampling points of superficial atmospheric sediment in the streets showed smaller mercury concentrations in the wet period between October and January where the minimum value was 109 µg.kg-1 in January and a maximum value of 1163 µg.kg-1 in October (Figure 3). The months from May through September that represent the dry period showed averages between 1526.64 – 1932.03 µg.kg–1.

Figure 3. Mercury concentration distribution in superficial atmospheric sediment throughout the months of 2003.

All the street dust collection points showed high levels of mercury; moreover, the variability was high between the sampling points and with time in 2003.

The atmospheric evaluation inside the five gold workshops in the commercial area of Porto Velho was carried out using the air bubbler system (Table 1).

The mercury concentration found in the workshops was low and the highest value found in the two collection periods was 0.0050 g/m3 of mercury.

Table 1. Total mercury concentration in atmospheric air inside gold workshops in Porto Velho-RO.

| Gold Shops | [Hg] (g/m3) Dry Season 2002 | [Hg] (g/m3) Dry Season 2003 |
| --- | --- | --- |
| A | 0.0050 | 0.0040 |
| B | 0.0010 | 0.0001 |
| C | 0.0001 | 0.0001 |
| D | 0.0001 | 0.0010 |
| E | 0.0030 | ------ |

In the occupational evaluation through urine samples the values of mercury ranged from 0.90 to 41.57µg.L-1, without exceeding the limit value recommended by WHO of 50.00 µg.L-1 (Table 2).

Table 2. Average Hg concentration in the urine of gold workshop workers before and after work in Porto Velho, 2002/2003.

| B.D.J. (g.L-1) | S.D. | Min. | Max. | E.D.J. (g.L-1) | S.D. | Min. | Max. | n |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.51 | 12.07 | 1.84 | 41.50 | 11.43 | 8.15 | 0.90 | 29.42 | 10 |

B.D.J.: Beginning of day’s journey               E.D.J.: End of day’s journey.

Conclusion

In the superficial atmospheric sediment from the streets 70.83% of samples showed values above the average mercury content of Amazon region soils (100-200µg.kg–1). The dry period (May – September) showed the highest concentration, especially due to the presence of gold mining activity during the dry period. This, consequently, increases the combustion activities in the gold workshops. The low values found in the atmosphere inside the workshops are a result of the use of hoods, thus reducing the Hg vapour concentrations inside the shops and releasing it into the open air, outside the shops. In the occupational evaluation none of values found exceeded the WHO values.

Acknowledgements

This work was supported by UNIR, PNOPG/CNPq No. 55.0882/01-4, RIOMAR Foundation and SEBRAE-RO.

Reference

Bastos, W.R.; Malm, O.; Pfeiffer, W.C. & Cleary, D. (1998). Establishment and analytical quality control of laboratories for Hg determination in biological and geological samples in the Amazon Brazil. Ciência e Cultura Journal of the Brasilian Association for the Advencement of Science,v. 50, p. 255-260.

Lindqvist, O.; Jernelov, A.; Johansson, K. & Rodhe, H. (1984). Mercury in the Swedish Environmental: Global and local sources. p. 16-46. Suíça.

Malm, O. (1991). Contaminação ambiental e humana por mercúrio na região garimpeira de ouro do rio Madeira, Amazônia. Tese (Doutor em Ciências)- Instituto de Biofísica Carlos Chagas Filho da Universidade Federal do Rio de Janeiro.

Pfeiffer, W.C.; Malm, O.; Lacerda, L.D. & Karez, C.S. (1992): Contaminação ambiental e humana por metais pesados: uma revisão. In: O Ambiente Inteiro, a contribuição critica da Universidade a Questão Ambiental. Ed. UFRJ, p. 184-194.