## Clean-up Strategies for Microbial Mercury Removal from Contaminated Environment

W.-D. Deckwer1, J. Leonhäuser2, S. Ledakowicz3, I. Wagner-Döbler2

1Technical University, Braunschweig, Germany

2 Technical University of Lodz, Poland

3National Research Institute for Biotechnology (GBF), Braunschweig, Germany

Mercury contaminations cover all phases: atmosphere, water and soil. In the global Hg recycle microbial conversions play an important role. The basic feature of the bacterial resistance mechanism is the conversion of ionic Hg to its elemental form under stoichionetric consumption of biochemical reduction equivalents (NADPH2). Therefore, any microbial remediation technique requires Hg resistant strains in an aqueous phase with the biomass acting as biocatalyst as well as provider of reductive power. Hence, contaminations of air (oft-gas from industrial activities, flue gas) and solids (deposites, river-sediments, back-construction contaminated sites) all need transfer into the aqueous phase by sorption or solubilization processes.

The design of industrial scale plants for Hg removal from effluent streams depends on various factors such Hg concentration, flowrate, pH, temperature, variations of inlet conditions, other loadings (salts, organic chemicals), pretreament steps,  and last but least the required outlet concentration. The central core of any demercurization process is the microbial reactor the choice of which is of utmost importance for the entire process design. Indeed, the achievable conversion (of ionic to elemental mercury) decisively depends on the reactor type and its volume which in turn is determined by the

- reaction rate law of the microbial transformation

- biomass concentration (freely suspended vs. immobilized)

- mode of operation (batch vs. continuous)

- masstransfer limitations

- hydrodynamics (residence time distribution) for continuous flow (no mixing vs. fully mixed)

- removal of product (Hg0).

At industrially relevant conditions the reaction rate follows a typical substrate (Hg2+) inhibition law and passes a maximum at about 1 mg Hg2+/l. Therefore, a combination of two reactors (stirred tank followed by plug flow) is the optimal choice with regard to space-time-yield (minimum reactor volume, low investment). However, other factors such as ease of operation, simple process design, remote control, robustness due to disturbances and flexibility towards loading variations must be considered as well. In the contribution, various reactor choices and process designs are elucidated and compared in view of their volume demand. In particular, continuously operated mercury removal processes with immobilized microorganisms are evaluated on the basis of comprehensive experimental data from an industrially operated fixed bed system (proven technology) and a lab scale fluidized bed reactor. The advantages and drawbacks of the two routes are worked out and cost comparisons (including conventional technologies such as ion exchange) are given.