Fishes as indicators of Hg availability in Amazonian aquatic systems affected by gold mining

Zuleica C. Castilhos1, Saulo Rodrigues-Filho1, Ana Paula Rodrigues1,3, Roberto C. Villas-Bôas1, Marcello M. Veiga2 , Christian Beinhoff2

1 CETEM, Centre for Mineral Technology, Av. Ipê, 900, 21941-590,

Rio de Janeiro, Brazil, zcastilhos@cetem.gov.br; 2 UNIDO, United Nation Industrial Development Organization, Vienna International Centre, P.O. Box 300, A-1400, Vienna, Austria; 3 Fluminense Federal University, Department of Geochemistry, Niterói, Brazil.

Abstract: The main objective of this work is to investigate and suggest indicator fish species for Hg availability in Amazonian aquatic systems affected by gold mining activities. A total of 234 fish specimens of 16 species were collected: 73 specimens belonging to 13 species in São Chico and 161 specimens of 11 species in Creporizinho. The correlation analysis between Hg levels and length or weight, by using non-transformed or log-transformed data, and also by using length intervals and mean Hg concentrations in muscles suggest that traíra may be a good fish indicator of Hg availability, at least, in Amazonian aquatic systems strongly affected by gold mining. In addition, aETCcari, Curimatã, Ituí, Piranha and Sairu are also good indicators for specific sites, suggesting that, in order to find a Hg contamination indicator organism, search for a site-specific fish species, more than one species for several sites, could be a suitable approach in tropical aquatic ecosystems .

Keywords: Amazonian fish, mercury, gold mining.

INTRODUCTION CAPITALISED according to sample abstracts

The present work describes part of the results achieved in two small scale gold mining areas in the Brazilian Amazon - São Chico and Creporizinho - as part of the environmental and health assessment (E&HA) conducted by the Centre for Mineral Technology (CETEM) with the collaboration of the Evandro Chagas Institute, under the general coordination of the United Nations Industrial Development Organization . The E&HA is a part of the GEF/UNIDP/UNIDO Global Mercury Project - Removal of Barriers to the Introduction of Cleaner Artisanal Gold Mining and Extraction Technologies.  The main objective of this work is to investigate and suggest an indicator fish species for Hg availability in Amazonian aquatic systems affected by gold mining activities  releasing Hg.

Results and Discussion

A total of 234 fish specimens of 16 species were collected: 73 specimens belonging to 13 species in São Chico and 161 specimens of 11 species in Creporizinho. A total of 7 common species could be collected in both areas (acari, cará, curimatã, mandi, piau, piranha and traíra). Mercury levels in fish from 11 sites belonging to two areas were investigated (refer to a figure with a map!): 4 in São Chico (A1-flooded open pits; A2-São Chico reservoir, A3-flooded open pit near Rosa stream; A4- Conrado inflow and Novo river) and 7 in Creporizinho (A5-Papagaio mining site; A6- Tabocal mining site, A7-Buriti mining site; A9- Porto Alegre, Crepori river; A10- Crepori river upstream of Creporizinho inflow; A11- Chico Chimango creek, clear water near the Creporizinho inflow into the Crepori river). It should be stressed that it was not easy to catch fishes near the garimpo’s areas, because the high turbidity from mining activities deteriorate living conditions for the fishes.

Fish from São Chico are  heavier and larger than those from Creporizinho area (p<0.0001), but showed no correlation between fish Hg concentration and weight or length when analyzing all specimens in both areas. The significant correlation between Hg levels and length is very low and the higher correlation between Hg levels and weight is not significant. Maybe it is because we are dealing with many species within each group. Additionally, in this work, the weight measurements were not performed for fish lighter than 100 g, and this parameter were performed just for 35% of the specimens collected (n=81).

Tables 1 and 2 show total mercury levels in several fish species, including Traíras and Carás from several sites of two study area, São Chico and Creporizinho. In São Chico area, all fish species were collected only in one site, or in very few number in more than one site, except traíras and carás.  The results showed that carás and Traíras from São Chico have higher mercury levels (2.211.28; n=23 and 6.115.93; n= 20, respectively) than from Creporizinho (0.550.35, n= 44; 0.800.55, n=10, respectively). Others species did not show any difference between areas. Besides these results, it should be considered that Traíras and Carás were present in almost all of study sites in these studied aquatic systems. The site-specific ratio of mercury between these species, a carnivorous and a non-carnivorous one, ranged from about 2 to 4, which means that Hg accumulates from 2 to 4 times more in Traíras than in Carás for all of sites studied. In addition, the results showed that Hg levels only in Traíras from A2 are higher than Traíras from the other sites, which are similar one each other (Kruskall-Wallis and Mann-Whitney U-Test). In contrast, mercury levels in Carás from A1 and A2 are higher than Carás from A5, A6, and A8 (p<0.005); Carás from A5 and A6 are similar each other and lower than levels in Carás from A8 (p<0.05). These results suggest that Carás could be a better indicator for spatial Hg contamination than Traíras, since they became apparent differences among distinct sites. However, Traíras and Carás did not show any correlation between mercury levels and length or weight when total data were analyzed in São Chico and/or Creporizinho and, even, when data from each site was considered. In Creporizinho area, Piranha specie showed higher Hg levels in specimens from A9 than from A11.

In São Chico area, the species Curimatã and Ituí showed positive and significant correlation between Hg and length (0.75, n=8 and 0.9, n=5, respectively). In Creporizinho area, there are significant correlation between Hg and length and Hg and weight with Curimatã’ and Piranha’ species. The Curimatã specimens showed positives and significant correlation between Hg and length (0.88; p<0.05, n=6) and between Hg and weight (0.87; p<0.01, n=5). The Piranha specimens showed strong positive and significant correlation between Hg and weight (0.786, n=10).

Correlation analysis for each species collected from different sites was performed when the number of specimens were more than 4. The results showed significant correlation between Hg and length only for Curimatã from A1 (0.75; n-=8) and Acari from A9 (0.74; n=14) and Piranha from A11 showed significant correlation between Hg and weight (0.86; n=6). Although Traíras and Carás did not show any correlation between mercury levels and length or weight it was investigated the correspondence between log of length and log of mercury levels in muscles (Veiga, 2003).

Table 1 -  Total Hg levels in  fishes species (g/g ) from São Chico garimpo area

| Popular name | Mercury (g/g) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | A1(n) | A2 ( n) | A3 (n ) | A4 ( n) |  |
| Arraia |  |  |  | 0.630.27 (3) |  |
| Lambari |  |  | 0.430.13 (4) |  |  |
| Piranha |  |  |  | 0.930.38 (2) |  |
| Pirarara |  |  |  | 0.27 (1) |  |
| Surubim |  |  |  | 1.2 (1) |  |
| Traíra | 2.6 (1) | 9.015.4(13) | 0.390.09 (5) | 0.60 (1) |  |
| Acari |  |  | 0.030.003 (2) |  |  |
| Curimatã | 0.160.04(8) |  |  |  |  |
| Pacu | 0.09 (1) |  |  |  |  |
| Piau | 0.14(1) |  |  |  |  |
| Ituí |  |  | 0.300.06(5) |  |  |
| Cará | 1.360.15(3) | 2.341.33 (20) |  |  |  |
| Mandi | 0.25 (1) |  |  | 1.6 (1) |  |

n= number of specimens

Table 2 - Total Hg levels in fish (g/g ) from Creporizinho garimpo area

| Popular name | Mercury (g/g) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A5 (n) | A6 (n) | A7 (n) | A8 (n) | A9 (n) | A10 (n) | A11 (n) |
| Bocudo |  |  |  |  |  | 0.266 (1) |  |
| Piranha |  |  |  |  | 0.440.15 (6) | 0.380.01 (2) | 0.250.06 (8) |
| Traíra | 0.610.29 (2) | 0.380.03 (3) | 1.540.78 (2) | 1.040.36 (2) |  | 0.46 (1) |  |
| Acari |  |  |  |  | 0.050.01 (11) |  | 0.040.02 (2) |
| Curimatã |  |  |  |  | 0.110.06 (3) | 0.15 (1) | 0.120.04 (2) |
| Piau |  | 0.19 (1) | 0.30 (1) |  |  |  | 0.060.02(13) |
| Sairu | 0.200.04 (10) | 0.200.06 (7) | 0.240.07 (29) | 0.51 (1) |  | 0.150.04 (3) |  |
| Cará | 0.250.06 (5) | 0.110.07 (3) | 0.27 (1) | 0.650.33 (35) |  |  |  |
| Candiru |  |  |  |  | 0.910.64 (2) | 0.560.07 (2) |  |
| Mandi |  | 1.3 (1) |  | 0.65 (1) |  |  |  |

N= number of specimens

The data analyzed showed very low correlation between parameters for all Traíras from São Chico and for Traíras from A2. The analysis were also performed with species from all sites, but there was strong correlation only for Curimatã from A1 (y=1.5x-3.94; R2=0.42, n=8), Acari (y=2.6x+6.68; R2=0.57, n=11 )and Piranhas from A9 (y=1.64x-3.86; R2=0.81, n=6). Considering the low correlation coefficient resulting from analysis of transformed data for most part of species studied, it was investigated the correspondence between length intervals (close to 10cm and/or 5cm) and the average mercury levels in fish muscles, trying to decrease the high variability in data, and considering data not log-transformed. In addition, these data were log-transformed and the correlation coefficient were also investigated. It is highlighted that, generally, there are no significant differences between the analyses, since the results showed the same tendency.

Traíras from A2 showed a significant positive correlation between Hg levels and length considering either, non log-transformed or log-transformed data (y = 1.3x – 2.11; R2 =0.6). We should consider that changes in intervals of length could change the linear relationship found for Traíras from A2. Nevertheless, one could suggest that Traíras can be used as indicator organism for mercury contamination, at least in Amazonian heavily contaminated sites. Inversely, Carás from A2, São Chico area, showed a strong negative correlation between length and Hg levels and Carás from A8, Creporizinho area showed very low liner relationship. Other fish species from Creporizinho and its sites were also tested, and the best results were for Acari from A9 (y = 0.01x – 0.06; R2 = 0.95), Sairu from A6 (y = 0.03x – 0.08; R2 = 0.56) and, finally, for Piranhas from A9 (y=0.05x + 0.28; R2=0.85).

Moreover, the relationship between mercury levels in fish and mercury in sediments can be used as an indicator of bioavailability of mercury in aquatic system. It was performed the ratio between mean mercury levels in fish and IGEO, which is a contamination classification system (Müller, 1979, as cited by Rodrigues-Filho, 2004). Traíra and Piranha (carnivorous) and Cará and Curimatã (non carnivorous) were chosen because they were collected in São Chico and Creporizinho areas and represent different food habits. All sites showed the ratio below of 0.5 considering all species, except A2 and A7 for Traíra, which resulted in 1.5 and 1.8, respectively. These results suggest that A2 and A7 showed the highest mercury bioavailability for fish from sediments.

Conclusions

Although Traira does not show good correlation between Hg and length, it should be used as a Hg bioindicator for the following reasons: it shows the highest Hg levels; it has the most widespread distribution in tropical fresh waters, it adapts easily to adverse conditions in impacted areas and, it is appreciated for human consumption. Acari, Curimatã, Ituí, Piranha and Sairu showed linear correlation between Hg levels and length or weight, suggesting that, in order to find a Hg contamination indicator organisms, search for a site-specific fish species, more than one specie for several sites, could be a better approach in tropical aquatic ecosystems. They could be used for temporal analysis rather than for spatial one. Moreover, although Carás did not show any correlation between Hg levels and length or weight in any site, they showed significant differences among distinct sites, suggesting they can be used as indicator organisms for spatial analysis.

References

Rodrigues-Filho, S et al. (2004): Environmental and Health Assessment in Two Small Scale gold mining areas-Brazil. São Chico and Creporizinho. Final Report.120 pp.

Veiga, M. M. & Baker, R. (2003): Protocols for Environmental & Health Assessment of mercury Released by Artisanal and Small-Scale Gold Miners (ASM). Global Mercury Project, UNIDO.