Kinetic and thermodynamic aspects of microbial reduction of mercury ions

S. Ledakowicz1, H. Danhamer2, U. F. Becker3 and W.-D.Deckwer2,4

1Technical University of Lodz, Poland

2National Centre for Biotechnology GBFmbH, Germany

3Degussa AG, Frankfurt/Main, Germany

4Technical University of Braunschweig, Germany

The technology of microbial removal of ionic mercury from wastewater of chlor-alkali amalgam process by means of bioreduction of ionic mercury to insoluble metallic mercury has been developed in GBF, starting from screening of mercury resistant microorganisms (with expression of mercuric reductase), through kinetic and thermodynamic studies of the process, employment of bioreactors with fixed-bed and fluidised-bed of immobilized microorganisms, up to its industrial implementation. The present paper focused on determination of thermodynamic and kinetic data of the system for the mercury bioremediation.

The ionic mercury Hg(II) biotransformation to volatile elemental Hg(0) is accompanied by transfer phenomena, which involved liquid-solid mass transfer of Hg(II), intra-particle diffusion and liquid-gas desorption of Hg(0) processes. The parameters describing these three mass transfer steps i.e. effective diffusivity of Hg(II) in biocatalyst-beads and mass transfer coefficients kS for Hg(II) and kLa for Hg(0) have been determined experimentally. Additionally experiments were performed to determine the necessary equilibrium data i.e. Hg(0) solubility in water, adsorption isotherm for Hg(II) into biocatalyst carrier and Henry’s constant for the system Hg(0) vapor/air - water. Transport resistance evaluation revealed that all resistances of mass transfer are significant and should be taken into account during modeling and designing of this promising mercury removal bioprocess.

The intrinsic kinetics of mercuric ions reduction by freely suspended microorganisms in metabolic active (viable cells), intact and permeabilized state, under various conditions was determined. The kinetic experiments were carried out in a specially designed magnetically stirred glass microreactor. The elemental mercury formed by the microorganisms was directly blown out from the reactor into atomic absorption spectrometer. As the mercuric reductase is dependent on cofactor, the NADPH was added together with -mercaptoethanol to mercury reduction assays. The rates of mercury reduction by viable and permeabilized cells are comparable, and higher than those for intact cells. The Michaelis-Menten formalism was employed to describe the kinetic data for intact cells, while for the viable and permeabilized cells the substrate inhibition had to be taken into account the Monod-Haldane equation. The observed strong inhibition of chloride ions on Hg (II) biotransfromation rate was explained taking into account the ionic mercury-chloride equilibrium (formation of HgCl3-and HgCl42-complexes under higher content of salts). From the comparison of kinetic data for the viable and the permeabilized cells one can conclude that mercuric ions biotransformation depends on metabolic activity of the microorganism, in particular on ATP-dependent transport proteins, whose activity is essentially reduced in the intact cells. The developed kinetic model can satisfactory predict the mercuric reduction rate in a wide range of Hg(II) concentration. The experimental verification of the proposed model can confirm the possible mercury reduction mechanism, which can contribute to better understanding of the mercury biotransformation process and to designing of a more effective biocatalyst.