Total Mercury Levels in Fish from Akosombo and Kpong

Hydroelectric Reservoirs in Ghana

D. A. Baah, R. B. Voegborlo, E. E. Kwaansa-Ansah, J. H. Ephraim, A. A. Adimado

Department of Chemistry, Kwame Nkrumah University of Science and Technology, Kumasi, Ghana.

ABSTRACT

Mercury is a contaminant of concern that is known to bioaccumulate through the food web. There is evidence that mercury causes damage to wildlife population and poses a threat to human health through the consumption of contaminated fish. The toxicity of mercury is influenced by its form, environmental media, environmental conditions, and the sensitivity or tolerance of the organism. In general, toxic effects occur because mercury binds to proteins and alters protein production or synthesis. Toxicological impairments include reproductive impairment, growth inhibition, developmental abnormalities, and altered behavioral responses.

Several studies worldwide have indicated considerable mercury levels in fish samples collected from hydroelectric power generation reservoirs. The increase in fish mercury levels was generally believed to be a consequence of bacterial methylation, to form methylmercury, stimulated by benthic conditions caused as a result of decomposition of flooded organic materials. Methylmercury, which can be absorbed directly from water across the gills or be obtained from food, is the form in which most of the mercury is concentrated in fish. Fish and seafood retain methylmercury, of which up to 90% can be stored in their body tissues and transferred to humans on consumption.

The construction of dams to create reservoirs is always associated with changes in ecosystems. The Akosombo and Kpong Hydroelectric Dams are located downstream of the Volta Lake, which is the largest man-made lake in the world and occupies 4% of the total land area of Ghana, was created to generate hydropower for industrial development. The large area of land drained by the Volta River and its tributaries could enhance the accumulation of pollutant in these reservoirs. In this study, 50 samples of fish (representing 9 species of fish) were collected from the Akosombo and Kpong Hydroelectric Reservoirs (the only two Hydroelectric Power Generation Stations in Ghana) and their total mercury levels measured using cold vapour atomic absorption spectrometry (CVAAS). The samples were digested with nitric-perchloric acid (1:1) followed by sulphuric acid and heating to about 200°C for 30mins prior to analyses.

The overall results for fish samples from the two reservoirs were in the range

0.003-0.126µgg-1 (wet wt.). The maximum level of 0.126µgg-1 (wet wt.) recorded in a Tilapia species of total bodyweight of 174.85g is over 200% lower than the WHO recommended guide of 0.5µgg-1 (wet wt.) for mercury in fish for human consumption. Thus, fish from these reservoirs could be said to safe for human consumption with respect to mercury.