PREDICTION OF MERCURY-BEARING GAS DEPOSITS

(THEORETICAL AND ENVIRONMENTAL ASPECTS)

N.A.Ozerova1, N.R.Mashyanov2, V.V.Ryzhov2, Yu.J. Pikovsky3

1 Institute of Geology of Ore Deposits, Petrography, Mineralogy and Geochemistry, Russian Academy of Sciences (IGEM RAS), Moscow, Russia, e-mail ozerov@rinet.ru;

2 St.-Petersburg State University, St.-Petersburg, Russia, e-mail nrm@lumex.ru;

3 Moscow State University, Moscow, Russia, e-mail ummgu@mail.ru

Abstract: Based upon many-year studies of mercury geochemistry conducted in the Institute of Geology of Ore Deposits, Petrography, Mineralogy and Geochemistry, RAS (Moscow), regional patterns of dislocation of mercury-bearing gas and gas-oil deposits have been established. This allowed working out certain methods for predicting such deposits.

Key words: mercury, gas deposits, prediction

Introduction

Prediction of mercury-bearing gas deposits is based upon the concept of Earth’s mercury degassing, when mercury (Hg) within the fluid flow is supplied from the Earth’s mantle along the deep fault zones in the periods of their or their separate units’ activity, and takes part in the formation of various deposits including gas and gas-oil ones. This allowed distinguishing mercury-ore belts of a new type. Here refers the distinguished ore belt of the Karpinsky lineament. Singling out of this structure appeared quite productive; increased Hg concentrations have been predicted and discovered in a number of gas deposits within its limits. This had been done for the gas-condensate field of Oposhnya (Dneprovo-Donetskaya depression, Ukraine), and for the Astrakhan gas-condensate field (Near-Caspian depression, Russia).

Methods of investigation. We used geological and geophysical data that allowed to make conclusions about the bedding depth of tectonic structures controlling the distribution of gas deposits, about the connection of these structures with the Earth’s mantle, as well as about their activation during the formation of deposits.

In the beginning of our studies, mercury contents in gasses from the wells (Oposhnya deposit, and others) was determined by means of pumping great volumes of mercury through the barbater with a known mercury absorbent, and was determined by turbo-colorimetric method using the Polezhaev’s reagent. Later, mercury content in gases from the wells, flue-gasses, gas-processing plant (Astrakhan deposit, and others) and atmospheric air, as well as in condensates, was determined by the specific atomic-absorption method worked out in Saint-Petersburg State University: Sholupov, Ganeev (1995). Sulfur was analyzed using the method based upon the abovementioned chemical method as applied to the sulfur of hydrocarbon deposits.

Results and discussion

Fig. 1. Scheme of dislocation of the Oposhnya Deposit

1 – crystalline massifs (A – Voronezhski, B – Ukrainian); 2 – Dneprovo-Donetskaya Depression and its southeastern extension – Donbass; 3 – transverse deep faults (VL – Verkhovtsevsko-Lgovsky); 4 – Central-Donetsky longitudinal fault; 5 – gas deposits and manifestations (a – with mercury content in gasses being < 1•10-6 g/m3, b – with that of - >1•10-6 g/m3): O – Oposhnyanskoye, S – Solokhovskoye, G – Gadyachskoye, M – Mashevskoye, Sh - Shebelinskoye; 6 – mercury deposits and ore manifestations (Sl – Slavinskoye, K – Konstantinovskoye, N – Nikitovskoye, V – Verkhovtsevskoye)

Fig. 2. Seismological profile across the Dneprovo-Donetskaya depression (longitudinal profile) and positions of gas deposits

1, 2 – sedimentary complexes: 1 – Upper Paleozoic and Mesozoic, 2 – Riphean and lower Paleozoic; 3 – crystalline basement; 4 – surface M; 5 – seismic margin of the upper and lower sedimentary complexes; 6 – deep fault zones; 7 – gas deposits: 1 – Gadyach, 2 – Oposhnya, Solokhovskoye, 3 – Mashevskoye, 4 – Shebelinskoye (mercury-bearing ones are darkened)

The Oposhnya deposit is located within the Dneprovo-Donetskaya depression that presents a fragment of the Karpinsky lineament. It is confined to a rather permeable unit of the Earth’s crust, that is, to the cross-point of the Central-Donetsky (Axial) longitudinal fault of mantle bedding (by the results of seismological studies) and the Verkhovtsevsko-Lgovskii transverse fault (Fig. 1, 2), which is favorable for mercury supply from the Earth’s mantle. Transverse faults of mantle bedding controlling the dislocation of the Oposhnya and Gadyach deposits (mercury-bearing, as it has been established) were activated during the Alpine Period, including Anthropogenic one (sub-meridional position of rivers along these faults, direct geophysical surveys of vertical and horizontal movements, powerful gas emissions in the mines of Krivoi Rog, etc.). Besides, the following seismoactive factor should be taken into consideration: the Oposhnya deposit is located in the epicenter of isolines of the proximity to strong earthquakes. This allowed considering the Oposhnya field as a very promising object in respect to its mercury-bearing capacity.

The above was confirmed by field research works. Hg content in well gases ranges from 1.5×10-5 to 1.1×10-2 g/m3 and increases with the depth of bedding of gas-bearing layers (for 1400 m), which agrees with the concept of the Earth’s mercury degassing. In the condensates of hydrocar-bonaceous gasses mercury content reaches 1.5×10-3 g/l. Accordingly, as recalculated for the gas-condensate mixture, average mercury content makes (1-3)×10-4 g/m3. We should bear in mind that over the area of the deposit mercury is supplied with gasses along the faults, pores and fissures, and can create local anomalies in the atmosphere, which, in their turn, may affect the environmental situation.

100

110

90

N

Astrakhan

а

100

piquet numbers

1

2

3

4

90

95

100

105

110

5,62/1,64

6,10/1,69

6,24/1,82

6,07/1,80

6,25/1,79

km

6,35/1,82

6,20/1,69

6,17/1,79

6,37/1,79

B

6,70/1,74

6,80/1,78

7,10/1,73

6,95/1,79

M

8,00/1,73

8,10/1,73

40

20

0

7,75/1,77

b

6,25/1,79

piquet numbers

а

б

6

1

2

3

4

5

7

9

8

10

Fig. 3. Deep structure of the Astrakhan deposit area - by Bulin and Egorkin (2000)

a – scheme of the position of the central sector of the geotraverse, Krasnodar – Emba River (Multiwave deep seismic profiling);

b – seismic profile of the Earth’s crust in the Astrakhan deposit area

a. 1 – Astrakhan deposit; 2 – oil manifestations; 3 – geotraverse line and piquet numbers; 4 – elements of linear ring and arc structures by the data of cosmic surveys’ decoding;

b. – 1-3 – seismic margins: 1 – serial; 2 - surface of the “seismic basement” (B), 3 – Mokhorovichich border (M); 4 – mid-layer velocity of longitudinal seismic waves, with Vp, km/s in the numerator and parameter of Vp/Vs in the denominator (VS – velocity of transverse seismic waves); 5 – conventional margins of domains with different velocity characteristics (velocity contacts); 6 – zones with increased values of Vp/Vs; 7 – zones with lower values of Vp/Vs; 8 – zones with increased velocity Vs: a – distinguished in the radial direction, b – those distinguished in the lateral direction; 9 – zones with lower velocity of seismic waves, distinguished in the radial direction; 10 – zones with lower Vs, velocity, distinguished along the lateral.

Astrakhan gas-condensate deposit is located in the upper part of the Astrakhan arch (Fig. 3). In the South, the Astrakhan arch and a nose of the crystalline basement corresponding to it at depth, border the Karakul step along the deep fault. This fault presents a marginal deep fault of the Near-Caspian depression included into the system of the Karpinsky lineament. Besides, the Astrakhan arch is broken by a large transverse failure – the Akhtubinsky deep fault traced along the Volga River course. Marginal longitudinal faults and the transverse failure of the Astrakhan arch preserved tectonic activity during the whole geological record (D-Q). Connection of activated faults with the deep layers of the Earth’s crust and mantle is evidenced by the results of deep seismic sounding, by increased density of the heat flow and temperature anomaly on the surface of the basement.

In order to evaluate Hg content within the gas-condensate mixture, we analyzed gas, condensate and sulfur separately. Hg content in gasses from the wells and in the separation gas at the Astrakhan gas-transfering plant (AGTP) varies from 0.3 to 3.5×10-6 g/m3. Hg content within condensates sampled at the AGTP makes up (0.8-3.6)×10-4 g/l. In most samples of sulfur, Hg content 1.7-2 and even 4 times exceeds its clarke; they range from 0.01 to 0.18 mg/kg. The above results show that the condensate mainly conditions mercury-bearing capacity of gas-condensate mixture. Knowing its amount in one cubic m of gas, we estimated Hg overall content in the gas-condensate mixture to be 5.4×10-5 g/m3, which is essentially increased concentration. By mercury contents in the gas-condensate mixture, Astrakhan deposit corresponds to quite a small-scale proper mercury manifestation. For estimating environmental impact of the processing of gas-condensate mixture from the Astrakhan deposit, tank gas, diesel and boiler fuel were also analyzed and showed rather low mercury contents. It should be stressed that mercury content in the tank gas does not exceed 0.12±0.009×10-6 g/m3, while the maximum concentration limit in Russia equals 0.3×10-6 g/m3. At the same time, gasoline (the product of condensate treatment) shows somewhat increased mercury content – 1×10-4 g/l, and this mercury is afterwards emitted to the atmosphere in the process of gasoline combustion. To estimate the possibilities of further transfer of mercury within the emissions from the tubes of the plant, flue gasses were analyzed with the purifying unit operating; in that case, mercury content was close to the abovementioned maximum concentration limit. Observations were carried out continuously for 1.5-2.5 hours with the plant operating in its normal mode. Measurements of mercury content in the atmospheric air in the area around AGTP have been made along all the highways, with the radius of about 2 km from the plant. The results obtained allow suggesting that environmental situation in the area of AGTP is quite favorable, and utilization of tank gas for industrial and domestic applications is environmentally safe.

- Conclusions

Data presented, as well as those earlier available: Ozerova (1986), evidence that mercury is an excellent marker, indicator of the participation of mercury bearing mantle fluids in the formation of gas deposits. This reflects an important theoretical (genetic) aspect of the results obtained. Besides, since mercury is one of the most active pollutants of the noosphere, these data are valuable for mercury monitoring during the extraction and processing of gas containing raw materials, as well as utilization of products of such processing.