# Survey of Land Contaminated with Mercury at PVC-SODA Factory in Vlora, Albania

Jaroslav Reif1, Jan Průša1, Vladimír Mejstřík2, Lubomír Procházka1, Ales Liška1, Pavel Bláha1, Roman Duras1, Jitka Novotná1, Eva Mátlová1, Ivan Dyba1,

Kujtim Biçaku3, Gazmend Gjyli4.

1GEOtest Brno, Czech Republic a.s., 2VÚOS a.s. Czech Republic,

3Environmental Institute, Albania, 4Ministry of Environment, Albania

reif@geotest.cz; prusaj@geotest.cz; matlova@geotest.cz

Abstract: The former chemical factory is located 4 km north of the city of Vlora at the sea coast. Mercury cathodes were used during its operation. This locality was included in hot spots in view of mercury pollution of coastal and sea ecosystems. The investigation was aimed at the determination of lateral and vertical distribution of mercury. Hg-concentrations were analyzed in ambient air, soil air, soils, groundwater, sea water, marine deposits and building materials. Hydrogeological, geochemical and geophysical survey and drilling were mainly used to achieve the goal. A multifunctional mercury spectrometer RA 915+ was applied for the analytical determination of mercury. The highest concentrations were detected near the electrolysis building and the road (tens of mg/kg). About 600 g of metallic mercury was separated from about 10 kg of electrolysis building material. Based on the geochemical survey it can be concluded that the mercury contamination in soils is bound to the unsaturated zone of the area of concern and decreases with depth (1-10 mg/kg at 1.5 m below ground); the mercury concentration in air in hot spots reaches values higher than 50 µg/m3; the mercury concentration in sea water at the coast line amounts to 0.9 – 3.4 µg/l; and the mercury concentration in sea deposits decreases from the sewerage outlet (2,010 µg/kg) to the distance of 500 m from the coast (50 µg/kg).

# Introduction

This report sums up results of the geochemical investigation carried out in 2002 to determine the areal and vertical distribution of mercury at the site of the former PVC – SODA factory in the town of Vlora (Valona) in Albania. The investigation focused on the determination of mercury concentrations in ambient air, soil air, soils, groundwater, sea water and marine deposits. To assess the mercury contamination of buildings, the investigation was also aimed at the determination of mercury concentration in building materials. Part of the accomplished investigation was to drill boreholes and to survey their elevations, to detect groundwater levels and groundwater flow directions, to stake out profiles of sampling points and to survey underground electric conductors. The area of concern comprises a part of the PVC – SODA factory and a part of the depository of lime sludge located between the factory site and the Adriatic Sea. The north-eastern boundary lies on the Vlora – Zvernec road; the south-eastern boundary is a wall separating the site of PVC production from the site of sodium carbonate production, and its imaginary continuation towards the sea. The boundary in the northwest is formed by the edge of a pine forest and its imaginary continuation towards the sea. The south-western boundary lies on the shoreline of the Adriatic Sea. During the investigation of contamination of marine deposits and sea water, the area of concern was extended to the shoreline. Contamination criteria for evaluation of results of chemical analyses are follows:

For ambient air mercury concentrations in the air, the limit values are given in Czech Republic Governmental Decree No. 350/2002 Coll. that stipulates immission (= ambient air quality) standards, and conditions and methods of the monitoring, assessment, evaluation and management of air quality, especially for health protection. This regulation is in full compliance with the European Union legislation.

Table No.1: Immission Standard Value for Hg

| Parameter | Time Interval | Limit Value [µg.m-3] | Admissible Number of Exceeding per Year |
| --- | --- | --- | --- |
| Hg | Calendar Year | 0.05 | 0 |

The issue of mercury concentrations in the working air is regulated by Czech Republic Governmental Degree No. 178/2001 Coll. that stipulates the conditions of occupational health protection of employees. The Decree defines the values of admissible exposition limit (PEL) and maximum admissible concentrations (NPK-P). The following limits are given for mercury: PEL = 0.05 mg.m-3 and NPK-P = 0.15 mg.m-3. The admissible concentrations of mercury in soil air are not legislatively regulated and defined. The evaluation of results of chemical analyses of soil samples and groundwater is performed according to the Methodological Instruction of the Ministry of Environment of the Czech Republic of 1996 to ensure the process of remediation of old ecological loads. This Methodological Instruction recommends criteria (A, B, C) of contamination of soils and groundwater for the needs of assessing ecological loads and deciding a further procedure in their solution, including loads being solved within privatization. The Methodological instruction stipulates individual criteria as limit concentrations of the given chemical substances in soil and groundwater. Comparison of concentration values detected in the investigation of contamination with such criteria allows to tentatively assess the level of contamination and to place such contamination into a category according to its seriousness. A further procedure for assessing contamination is recommended for each category.

These criteria, in case of groundwater, are applied only to selected metals and not to the other (e.g. inorganic) main ions, because it is necessary to take into account natural concentrations of such ions in the given environment. Table No.: 2 sum up criteria for soil parameters that were monitored within the performed geochemical investigation and Table No.: 3 for groundwater parameters. The area of concern is expected for industrial use.

Table No. 2: Overview of Criteria – Soils

| Criterion | As | Cd | Pb | Hg | Zn | Cr | Cu | Ni |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [mg.kg-1] | [mg.kg-1] | [mg.kg-1] | [mg.kg-1] | [mg.kg-1] | [mg.kg-1] | [mg.kg-1] | [mg.kg-1] |
| A | 30 | 0.5 | 80 | 0.4 | 150 | 130 | 70 | 60 |
| B | 65 | 10 | 250 | 2.5 | 1,500 | 450 | 500 | 180 |
| Cpr | 140 | 30 | 800 | 20 | 5,000 | 1,000 | 1,500 | 500 |

Table No. 3: Overview of Criteria – Groundwater

| Criterion | As | Cd | Pb | Hg | Zn | Cr | Cu | Ni |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [µg.l-1] | [µg.l-1] | [µg.l-1] | [µg.l-1] | [µg.l-1] | [µg.l-1] | [µg.l-1] | [µg.l-1] |
| A | 5 | 1.5 | 20 | 0.1 | 150 | 3 | 20 | 20 |
| B | 50 | 5 | 100 | 2 | 1,500 | 150 | 200 | 100 |
| Cpr | 100 | 20 | 200 | 5 | 5,000 | 300 | 500 | 200 |

# Geochemical Measurements

Mercury concentrations in the ambient air were determined in two height levels, 0.15 m and 1.0 m. As a whole, 662 measurements were performed on 331 measurement points. The measurement level of 0.15 m above ground was conducted at air temperatures t = 26 – 42 oC. The measured concentrations of mercury vapours were plotted in a map.  From the given map there is evident an area of increased mercury concentrations, the centre of which is the vicinity of Points 13.6, 13.7 and 13.5, i.e. the area near the electrolysis building (Object No. 626). This area is extended in the NW – SE direction. The mercury concentration was exceeded in Points 13.6, 13.7 and 12.5 and amounted to 30,000 ng.m-3 (30 g.m-3). The second hot spot, having concentrations exceeding 25,000 ng.m-3, is around Point 8.10, which lies in the proximity of the demercurisation station building (Object No. 815). Anomalies of lesser extent in terms of their area were found in the proximity of Point 12.16 (1,939 ng.m-3) to the NW from the anomaly of the electrolysis building and by the PVC–SODA – Zvernec road, where the concentration in Point 17.7 reached 3,768 ng.m-3. The measurement of mercury concentrations in the air at a height of 1.0 m confirmed the existence of the anomaly around Points 12.5 and 13.6 (Object No. 626), where the concentrations fluctuated above 20,000 ng.m-3. The concentrations exceeding 1,000 ng.m-3 were also detected at the site of the demercurisation station (Object No. 815). Increased concentrations were detected by the PVC–SODA – Zvernec road (Point 17.8), where the concentration reached 1,857 ng.m-3. Increased concentrations were also detected around Points 11.17 and 12.16. The measured concentrations (191 ng.m-3 and 118 ng.m-3) exceed neighbouring average concentrations about 5x.

The measurement of mercury concentrations in soil air was conducted in the identical network of points as in the measurement of mercury in ambient air. Where it was impossible to sample soil air from a depth of about 0.3 – 0.5 m, the position of such a measurement point was changed and planimetrically surveyed. The measurement was not conducted on profile no. 1 because of the high groundwater table level and on profile no. 18 because of its close proximity to the road embankment (soil pits could not be dug). The measured values of mercury concentrations in soil air were plotted in a map. From the map output there is evident the existence of the main anomaly bound to Points 12.7, 12.8 and 13.8 by the electrolysis building (Object No. 626), where the concentrations were higher than 30,000 ng.m-3. Another anomaly detected in Points 10.6 and 10.7 is found between Objects Nos. 648 and 622 by a road on the site boundary. Another anomaly, which lies at the site of the factory, is located in Points 17.6 and 17.7 by the PVC-SODA – Zvernec road. Point contamination was detected in the north-western area of the site in Point 11.17, where the concentration reached about 36,000 ng.m-3. In the area of the sludge depository there was detected an anomaly extending in the W direction from Point 8.7 down to Point 2.15, where the mercury concentrations steadily exceeded 10,000 ng.m-3, and point concentrations higher than 50,000 ng.m-3 were also detected. This anomaly also runs into a part of the area of the demercurisation station (Point 8.10).

In determining mercury concentrations in soils, we proceeded from the atmogeochemical investigation that was conducted in the entire surface of the area of concern. In determining mercury in soils, our attention was now focused only on the areas with anomalously high mercury concentrations in soil air. Soil samples were taken in the points of soil air measurement, and on the individual profiles in such a way so that the course of the entire anomaly could be encountered. Soils were collected only from the unsaturated zone, from depth levels of 0.0 m, 0.5 m, 1.0 m, 1.5 m, and, if possible, 2.0 m. A manual pedological probe and a manual earth auger were used for sampling. Soil samples were laid in airproof sample containers and transported to a workplace of a detached laboratory and analysed for mercury content. On the basis of soil analyses and areal distribution of mercury, the area of concern was formally divided into three areas: Area I – the PVC factory site proper, which is most contaminated in terms of area; Area II – the territory delineated by the PVC site and the Adriatic Sea, where lime sludge was dumped; Area III – the territory delineated by Area I and the boundary of the pine forest, where waste was dumped. The maps of Hg concentrations in soils have displayed the areas with concentrations higher than 20 mg.kg-1 and the areas with concentrations higher than 75 mg.kg-1. The areal extent of contamination in the individual depth levels is illustrated in Table No.: 4.

Table No. 4: Areal Extent of Hg Contamination in Five Depth Levels

| Depth | Area I |  | Area II |  | Area III |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | c > 20 mg.kg-1 | c > 75 mg.kg-1 | c > 20 mg.kg-1 | c > 75 mg.kg-1 | c > 20 mg.kg-1 | c > 75 mg.kg- |
| [m] | [m2] | [m2] | [m2] | [m2] | [m2] | [m2] |
| 0.0 | 41,600 | 17,680 | 5,500 | 60 | 1,340 | - |
| 0.5 | 17,800 | 2,260 | 1,580 | 120 | 2,000 | - |
| 1.0 | 6,400 | 175 | 6,320 | 880 | 240 | - |
| 1.5 | 5,350 | 200 | 1,320 | - | - | - |
| 2.0 | - | - | 240 | - | - | - |

The data on the areal extent of contamination will serve for the estimate of the volume of contaminated soils to be excavated in dependence on the fixed remediation limit. It arises from Table No.: 4 that in Area I there are about 25,175 m3 of soil with mercury contamination higher than 20 mg.kg-1, in Area II about 6,105 m3 and in Area III about 1,455 m3. In the course of the geochemical investigation, a building activity started in Area II (the sludge depository) with the subsequent excavation of lime sludge and the uncontrolled transport of the excavated material to the vicinity of the PVC factory site. For this reason, the state given in the maps is not relevant to the current situation but is valid for the period of the realised investigation. The detected vertical distribution of mercury in the area of concern reveals that the massive mercury contamination is bound to the unsaturated zone and significantly decreases with depth. Concentrations in the upper horizon of soils range on the order of X0.0 to X00.0 mg.kg-1, in places even up to X000.0 mg.kg-1. Mercury concentrations in soils at a depth of about 1.5 m below ground range on the order of 0.X – X.0 mg.kg-1. The higher concentrations are bound to the hot spot of contamination in the proximity of the electrolysis building and the road, where the concentrations range on the order of X0.0 mg.kg-1. The same situation concerning the vertical distribution of mercury was described in a study dealing with an investigation of the PVC factory site. Samples of marine deposits from the bottom of the Adriatic Sea were taken to assess a potential mercury contamination of the sea. Marine deposits were sampled from the sea surface by using a universal sampling device of the company Eijkelkamp for taking bottom sediments. A total of about 250 g of marine deposits was collected. The samples of deposits were put in sample containers and transported to a laboratory. The samples were analysed in the detached laboratory of the company GEOtest BRNO, a.s. in Vlora. A total of 87 samples was taken. These results were plotted in a map. The plotting of mercury concentrations in profiles E and G is given in Figure No.: 2. In it we can see a rapid drop in Hg concentration with increasing distance from the shoreline. The marked drop is evident approximately to a distance of 300 m from the shore; the drop in concentration is not so sharp at a larger distance. The concentrations range in the first tens of µg.kg-1. In profile X, which runs parallel to the shoreline at a distance of about 600 m, the detected concentrations are relatively the same, ranging between 34 and 46 µg.kg-1. The results reveal that the highest mercury concentrations were determined in Points E.1 (2,010 µg.kg-1) and F.1 (1,933 µg.kg-1), which are situated in the proximity of the sewerage outlet from the PVC-SODA factory. The course of mercury concentrations on the shoreline (d = 0 m) is displayed in Figure No.: 1, where also the position of the sewerage outlet is schematically represented. The mercury concentration in deposits at a distance of about 175 m from the shore is represented in the graph by line d = 175 m. The mercury concentrations decrease both in the direction parallel to the shoreline and in the direction perpendicular to the detected maximums.

Figure No. 1: The Mercury Concentration Profile of the Shoreline

Figure No. 2: The course of Hg Concentrations in Marine Deposits in profiles E and G

Sea water samples in contact with bottom sediments were taken in the same network of profiles as the marine deposits. Sea water samples were collected at a level of about 0.15 m above the bottom by means of a sampling device for zone sampling. After sampling, the samples were fixed with HNO3. The given table reveals that higher mercury concentrations are at a level of the shoreline between Points C.1 to G.1. The highest concentration (3.35 µg.l-1) was detected in Point C.1, which is situated approximately opposite well HM-3.

The quality of groundwater (Table No.: 5) was monitored in the first stage of investigation by sampling shallow wells HM-1 to HM-5 (07/2002). Groundwater samples were taken in the static state of the water-bearing layer. Samples for the determination of heavy metals were fixed with HNO3. Simultaneously with groundwater sampling, two sea water samples were taken (M-1, M-2). The quality of groundwater in the area of concern is very variable. Water from wells HM-2 and HM-3, which is strikingly basic, differs by pH value. The pH value of the other water has approximately a neutral character.

Table No.5: Physical-chemical parameters of water from the water wells (sampling - 23.7.2002).

|  | M-1 | M-2 | HM-1 | HM-2 | HM-3 | HM-4 | HM-5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 8.1 | 8.1 | 7.3 | 11.1 | 11.8 | 7.4 | 7.4 |
|  [µS.cm-1] | 52,100 | 52,000 | 2,510 | 13,760 | 6,520 | 1,978 | 785 |

Results of the shortened chemical analysis of sea water (M-1, M-2) and groundwater taken on the day of 23.7.2002. Abundance of the individual main ions according to the amount of substance and equivalent concentrations is summed up in ternary diagram. Sodium ions and chlorides rank among the most abundant ions; the least abundant are potassium and magnesium ions. In case of HM-1 sample, it is the water of SO4-Na molar type, HM-2 and HM-3 samples represent the water of Cl–Na molar type, and HM-4 and HM-5 samples are of mixed Cl-Na-Ca molar type. The relative abundance of main groups of ions of sea water and groundwater is displayed in a triangular diagram of Figure No.: 3. This graph represents the ratio of (Na+K):(Ca+Mg):(Cl+Mg) expressed in percents of the sum of equivalent concentrations (zi.ci). HM-3 and HM-5 water samples strikingly differ from the other water by their relative abundance of main groups of ions, namely by their higher percentage abundance of (Ca+Mg) group.

Figure No. 3: The relative abundance of groups of main ions in % of zi.ci
                                                         [(Na+K)+(Ca+Mg)+(Cl+SO4) = 1]

Concentrations of the selected heavy metals are given in Table No.: 6, in which the bold-faced values denote the exceeding of Criterion C for groundwater.

Table No. 6: Concentrations of Heavy Metals in Groundwater (sampling - 23.7.2002)

| Parameter | HM-1 | HM-2 | HM-3 | HM-4 | HM-5 | HM-6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Concentration [µg.l-1] |  |  |  |  |  |
| As | 14.6 | 24.0 | <5.00 | 6.00 | 31.2 | 12.0 |
| Cd | <5.00 | <25.0 | <10.0 | <1.00 | <1.00 | <1.00 |
| Pb | 285 | <250 | <100 | 39.3 | 40.6 | 65.5 |
| Hg | 2.80 | <1.00 | 47.0 | <1.00 | <1.00 | <1.00 |
| Cu | 206 | <250 | <100 | 20.3 | 17.5 | 27.0 |
| Zn | 223 | <500 | <200 | 86.6 | 44.2 | 58.7 |
| Cr | 174 | <250 | 104 | 119 | 76.1 | 133 |
| Ni | 334 | <500 | <200 | 287 | 160 | 266 |

Groundwater in the proximity of the electrolysis building (HM-1) exceeds Criterion C in the parameter of nickel and lead. Criterion C for mercury is exceeded in well HM-3, located in the lime sludge depository. Nickel is exceeded in water from well HM-4 at the factory site and from well HM-6, located at the edge of the forest.

In December 2002, a sample of sea water was taken at the shoreline and a sample from the water main shaft flowing opening of drinking water at the factory site, which supplies the first water-bearing layer (table No.: 7).

Table No. 7: Concentrations of Heavy Metals in Surface Water

| Parameter | M-1 (23.7.02) | M-2 (23.7.02) | The Sea (1.12.02) | Water Main Shaft (1.12.02) |
| --- | --- | --- | --- | --- |
|  | [µg.l-1] |  |  |  |
| As | < 5.00 | < 5.00 | < 5.00 | < 5.00 |
| Cd | 34.1 | < 25.0 | < 5.00 | < 1.00 |
| Pb | < 250 | < 250 | < 50.0 | < 10.0 |
| Hg | < 1.00 | < 1.00 | < 1.00 | < 1.00 |
| Cu | < 250 | < 250 | < 100 | 21.2 |
| Zn | < 500 | < 500 | 144 | < 20.0 |
| Cr | < 250 | < 250 | < 50.0 | < 10.0 |
| Ni | < 500 | < 500 | < 100 | < 20.0 |

The sampling of engineering structures was aimed at providing data on the degree of contamination of engineering structures of the individual buildings. Due to the state of the buildings, the sampling was performed from floor structures, structures of vertical components (walls and columns) at a level of 0.5 m above the floor, and from the building debris that was situated at the site. The samples of building material were taken from the floor surfaces or the vertical component to the maximum depth of 3 cm. The samples were placed in airproof sample containers and transported to the detached laboratory in Vlora. The inhabited buildings were not sampled. High mercury concentrations were detected in the material of the electrolysis building (Object No. 626), in which the mercury concentrations were higher than is the measurement range of the RA 915+ mercury analyser, and the samples were not measurable by the given equipment even after being diluted with inert material. Mercury concentrations in the building debris from the electrolysis building were determined by weight due to the macroscopic presence of mercury. About 10 kg of the building debris were taken from the floor of the building. This material was washed out, metallic mercury was separated by gravity on a pan and the separated metallic mercury was weighed. About 600 g of metallic mercury was separated from about 10 kg of the material.

3. Conclusion

On the basis of the accomplished geochemical investigation, it can be stated that the mercury contamination in soils is bound to the unsaturated zone of the area of concern: the thickness of the unsaturated zone ranges between about 1.2 m and 2.0 m; the mercury contamination in soils decreases with depth, and at a depth of 1.5 m below ground mercury occurs in units of mg.kg-1; the estimate of the volume of soils with concentrations higher than 20 mg.kg-1 amounts to 32,750 m3 in the area of concern; the mercury concentrations in the air in contamination hot spots are higher than 50 µg.m-3; the groundwater can be classified as being of the molar types SO4-Na (well HM-1) and Cl-Na (wells HM-2 and HM-3), and of mixed molar type Cl-Na-Ca (wells HM-5 and HM-6); the mercury concentrations in groundwater meet Criterion B of the MP MŽP ČR, except for well HM-3 (sampling of 07/2002); the groundwater has higher concentrations of lead, as a rule higher than 100 µg.l-1 (Criterion B), the mercury concentrations in sea water at a level of the shoreline range between 0.9 and 3.4 µg.l-1; the concentrations at a distance of about 50 m from the shoreline are < 0.1 µg.l-1; and the mercury concentrations in marine deposits decrease from 2,010 µg.kg-1 at the site of the sewerage outlet to 50 µg.kg-1 at a distance of about 500 m from the shore.