Arsenic and Heavy Metal Contamination and Human Health Risk Assessment in the Vicinity of the Abandoned Dongjung Au-Ag-Cu Mine, Korea

EunHye Chung1, Jin-Soo Lee1, Hyo-Taek Chon1*, Manfred Sager2

1School of Civil, Urban and Geosystem Engineering, Seoul National University, Seoul, Korea

2Austrian Agency for Health and Food Safety and Federal Office for Food Safety, Vienna, Austria

The purpose of this study is to investigate the contamination levels and geochemical behavior of heavy metals, and to assess human health risk around the abandoned Dongjung Au-Ag-Cu mine site in Kore. The environmental impacts are due to tailings and rock files of Au mining. There are residential land and farmland along the stream near to the mine and rock files.

Samples of soil and plant, tailing, soil, crop plant and water were collected in October, 2002 and in May, 2003. After appropriate preparation, soil and plant samples were analyzed for 22 main and trace elements by ICP-AES and ICP-MS. Sodium and K in water samples were analyzed using AAS, and other cations were by ICP-AES. Anions were analyzed by IC.

Concentrations of 0.011 As mg/L, 8.4 Mn mg/L, 5.6 Zn mg/L and 258 SO42- mg/L were found in stream water collected nearby waste rock piles. Arsenic concentration in groundwater, which has been used as drinking water, was 15.4 µg/L. Significant levels of heavy metals in tailings can impact on soils and waters around the mine. Mean concentrations of heavy metals in farmland soils were higher than those in uncontaminated soil. These metals were highly accumulated in plants grown at farmland soil. The highest level of heavy metals was found in bean leaves from the mine area (5.13 As mg/kg, 2.04 Cd mg/kg, 5.24 Cu mg/kg, 536 Mn mg/kg, 4.26 Pb mg/kg and 598 Zn mg/kg). Especially bean leaves had more 1.6 times of Cd than soil. The concentrations of heavy metals extracted by aqua regia are presented in Table 1.

Table 1. Range and average concentration of heavy metals in soil and tailings (unit in mg/kg).

| Sample Type | As | Cd | Cu | Mn | Pb | Zn | Hg |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tailings (n=5) | 2724 - 6688 | 1.7 - 16.5 | 58 - 756 | 2310 - 26063 | 5273 - 32631 | 528 - 5387 | 0.95 - 4.37 |
|  | 4074 | 5.9 | 377 | 7764 | 19150 | 2593 | 2.77 |
| Mountain soil (n=6) | 65 - 5262 | 2.9 - 152 | 28 - 224 | 1032 - 110125 | 119 - 12780 | 605 - 18388 | 0.06 - 6.08 |
|  | 1744 | 33.9 | 111 | 30431 | 4899 | 5117 | 2.06 |
| Farmland soil (n=30) | 28 - 1136 | 0.6 - 7.7 | 15 - 66 | 754 - 13875 | 72 - 1285 | 116 - 1812 | 0.05 - 2.15 |
|  | 115 | 1.6 | 31 | 2444 | 294 | 306 | 0.29 |

(n: number of samples)

* correspondence : chon@snu.ac.kr

For human health risk assessment, finding pathway of toxic elements to human body, exposure assessment, dose-response assessment and calculating risk for toxic and carcinogenic risk were performed. There are three possible pathways of heavy metals to human body; through mouth (oral exposure), through nose (inhalation exposure), and through skin (thermal exposure). In this study, only the oral route was considered for human health risk assessment. The exposure target was applied to the female resident of the mine area, and the scenario was assumed that she ingests heavy metals through rice, drinking water, and soil during her field farming.

Average daily dose (ADD) of toxic heavy metals was calculated by following equation.

C: Concentration of metals in soil, plant and water [mg/kg/day]

IR: Ingestion Rate [kg/day]

EF: Exposure Frequency [day/yr]

ED: Exposure Duration [yr]

BW: Body Weight [kg]

AT: Average Time [yr]

For the toxic risk, the hazard quotient (HQ) was calculated from ADD dividing by reference dose (RfD) for each element, and the hazard index (HI) was sum of all HQ. Carcinogenic risk was calculated by multiplying ADD with slope factor (SF). HQ indicates whether toxic elements can make adverse effect on health (HQ>1) or not (HQ<1), and carcinogenic risk means the probability of cancer occurrence. RfD and SF were quoted from IRIS (Integrated Risk Information System) database (U.S. EPA.).

Digestible contents of heavy metals in soil and plant were measured by EHS (Extraction of Heavy metals in Stomach and Small intestine) test for risk assessment. Because the values from aqua regia digestion of soil and total extraction of plant are the levels of soil and plant themselves, those values cannot indicate the proportion of available form. In EHS test, simulation of the stomach and the small intestines was performed. The test consists of 2-step extraction for 1 g of -80 mesh dried soil; 1 ml pepsin solution was added with 19 ml glycin buffer (pH 1.5) at 1st step, and 1 ml pancreatin solution with 19 ml alkaline buffer (pH 8) at 2nd step. Soils were shaken in bath for 100 rpm at 37℃ without any light input. For 1st and 2nd steps, the reaction was continued for 2 hours and overnight, respectively.

As the result of EHS test, the digestible concentrations of heavy metals in farmland soils are 11.1 As mg/kg, 1.0 Cd mg/kg, 8 Cu mg/kg, 487 Mn mg/kg, 71 Pb mg/kg, 128 Zn mg/kg, as shown in Fig. 1.

The calculated ADD and the result of health risk assessment are shown in Table 2.

Table 2. Health risk through oral exposure.

|  | Elements | Concentration | Concentration |  | Concentration |  | ADD | Oral risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | in soil (mg/kg) | in water (µg/L) |  | in rice (mg/kg) |  | (mg/kg/day) |  |
| Non - Carcinogenic effects | As | 11.1 | 6.44 |  | 0.63 |  | 5.10E-03 | 17.0 |
|  | Cd | 0.98 | 1.47 |  | 0.38 |  | 2.93E-03 | 2.98E+00 |
|  | Cu | 8 | 21 |  | 1.8 |  | 1.44E-02 | 3.59E-01 |
|  | Mn | 487 | 13 |  | 86 |  | 6.59E-01 | 4.71E+00 |
|  | Zn | 128 | 204 |  | 49 |  | 3.84E-01 | 1.28E+00 |
|  | SUM |  |  |  |  |  |  | 26.3 |
| Carcinogenic effects | As | 11.1 |  | 6.44 |  | 0.63 | 1.98E-03 | 3E-03 |

As a result of human health risk assessment from EHS test for determining digestible contents of heavy metals, there was a potential of cancer to 3 persons in one thousand due to oral exposure of As. Toxic heavy metals (As, Cd, Cu, Mn and Zn) could cause adverse effects on human health for the female residents of the mine area.