Springtime Mercury Depletion Events Observed in The Norvegian High Arctic,  Ny-Alesund, Spitsbergen

Sprovieri, F.1, Pirrone, N. 1, Cofone, F. 1, Stevens, R.2, Landis, M.3

f.sprovieri@cs.iia.cnr.it

1 CNR-Institute for Atmospheric Pollution, 87036 Rende, Italy

2 Florida Dept. Environmental Protection, NERL, USEPA, Research Triangle Park, NC 27709

3 NERL, USEPA, Research Triangle Park, NC 27709

Abstract: During an intensive mercury measurement campaign, carried in the Spring 2003 in Ny-Alesund, it was found that mercury concentrations in the lower atmosphere dropped in tandem with ozone levels. Hg0(g), Reactive Gaseous Mercury (RGM) and Hg (p)  concentrations were continuously measured at both the Italian research station (about 9 m a.s.l.), and on Zeppelin mountain, (474m a.s.l.) using Tekran system (2537A-1130-1135). Hg0 concentrations ranged between 2.6 ng m-3 to below the detection limit during the Mercury Depletion Events (MDEs) and then returned to typical levels, with an average of 1.3 ng m-3±  0.4 ng m-3. Our RGM data show an opposite trend during the study period with sharp increase in RGM concentrations up to 220 pg m-3, following the same trend as Hg0(g) depletion.  The observations at Ny-Alesund suggest that the depletion of Hg0 in the lower atmosphere observed after polar sunrise, is photochemically driven by oxidation of boundary-layer Hg0 probably by reactive halogens and that the springtime phenomenon creates a rapidly depositing species of oxidized gaseous mercury in the remote Arctic air at concentrations up to more than 220 pg m-3 .

Keywords: mercury, Arctic, depletion, atmosphere, measurements

# Introduction

Mercury exists in ambient air predominantly in the zero oxidation state as gaseous elemental Hg (Hgo) and comprises 97-99% of the total mercury found in the atmosphere with an estimated mean global tropospheric residence time on the order of 1 year1,2 allowing sufficient time for long-range atmospheric transport. Hg0 is, therefore, globally distributed by air mass circulation including remote environments such as the Arctic and the Antarctic. It has been found that during the polar spring elemental mercury may be converted to a reactive form that accumulates in the biota in the Arctic and Antarctic environment. These unusual springtime phenomena, called mercury depletion events (MDEs), is an important removal pathway for atmospheric mercury, therefore, a better understanding of them could lead to an improved description of global mercury-cycling. Hg depletion events have been observed annually at maritime circumpolar stations in both the Arctic: Alert, Ellesmere Island (Schroeder at al. 1998; Lu et al., 2001) Barrow, Alaska (Lindberg et al., 2002), and in Antarctica, Terra Nova Bay, Victoria Land (Sprovieri et al. 2002) after polar sunrise contemporaneously with tropospheric ozone depletion suggesting that both species were removed by similar unknown homogeneous and/or heterogeneous oxidation reactions. Episodic depletions of Hg0, closely resembling ozone depletions in Arctic surface level air, were frequently observed during an intensive atmospheric mercury campaign performed at Ny-Alesund, in the Norwegian high arctic (Svalbard Islands, Spitzbergen) from April 18th to May 12th 2003, during the arctic springtime. Hg0 concentrations dropped from around 1.7ngm-3 to undetectable levels (  0.1 ng m-3) and were strongly correlated with surface ozone (O3) depletion. The aim of this study was to present and discuss the results obtained during the Arctic spring campaign 2003 proposing a possible oxidative transformation process which converts Hg0 vapour into one or more much less volatile Hg(II) species.

## Methods

Springtime highly time-resolved measurements of atmospheric mercury species have been performed at Ny-Alesund, Svalbard Islands (78°54’N, 11°53’E) from 18th of April to 12th of May, 2003 in the presence of 24-h daylight. Mercury measurements were conducted at the Norwegian monitoring station on Zeppelin mountain (78°54’42”N, 11°55’30”E), 474 m above sea level. Hg0, RGM, and Hg(p) concentrations were automatically measured with 5 minutes and 2 hours time resolution respectively; in addition, 5-minutes time resolution ozone levels were also collected. Mercury species were simultaneously quantified by an automatic integrated system (a Tekran automated unit, Model 1130 and a Tekran 1135 particulate unit, coupled to an automated gas-phase mercury analyser, Tekran 2537A) which has the ability to concurrently monitor both elemental (GEM), reactive gaseous mercury (RGM) species and Hg(p) in ambient air at the picograms per cubic meter level (Landis et al., 2002). While the RGM is collecting on a quartz KCl-coated annular denuder warmed at a temperature of 50°C, Hg0  passes through at a flow rate of 1.5 L/min, with laminar flow and is collected at 5 minute intervals on a pair of gold traps in the Tekran 2537 analyser. While one trap is collecting the Hg0, the second trap is heated  at 500°C and the mercury released is analysed by Cold Vapour Atomic Fluorescence Spectrometry (CVAFS) (Munthe et al. 2001). The RGM collected on the KCl-annular denuders is thermally desorbed at 500°C to decompose and reduce RGM to the elemental form for the analysis step by the Tekran 2537A CVAFS downstream of the denuders. The Tekran 1135 particulate pyrolysis unit was used to determine Hg(p) levels by a quartz regenerable filter after the passing of the sample through the impactor downstream the denuders to eliminate coarse particles and pyrolyzer for airborne particulate mercury species. The quartz filter was desorbed at 800 C. During the filter heating step, the pyrolyzer was maintained at 800 C to ensure complete decomposition of all Hg(p) compounds evolved during the filter temperature ramp-up to Hg0. The total flow rate was 10 L.p.m. and the detection limit under the operationg conditions used was  2 pg m-3.

Results and Discussion

Three GEM depletion events are clearly recorded during the springtime campaign as shown in Figure 1, which shows the temporal trend of both GEM and O3 levels. O3 depletion is synchronized with Hg0 depletion. Hg0 concentrations ranged from background levels to undetectable values (< 0.1 ng m-3) in connection with the MDEs and then returned to typical levels, sometime exceeding global background. In particular, during the first major depletion event, depicted from 21th to 22th of April, GEM levels dropped very fast within 24 hours and the concentrations changed from about 1.7 ng m-3 to below 0.006 ng m-3, while in the following two minor depletion episodes GEM dropped below 0.25 and 0.5 ngm-3 respectively. It is noteworthy that MDEs coincide with polar sunrise ozone depletion events (ODEs) in the polar troposphere where ozone is destroyed by a photochemically reactions between ozone and reactive gaseous halogen species (especially bromine). These events cannot, in fact, be explained by changing meteorological parameters alone but coincide prevalently with the “bromine explosion” phenomena occurring especially close to abundant frozen and sea-ice surfaces. Bromine and chlorine radicals, in fact, are produced auto-catalytically from a heterogeneous photochemical mechanism involving sea-salt aerosol (Pirrone et al., 2003; Hedgecock et al., 2003, Sprovieri et al., 2003). During the ODEs, Hg0 exhibits a strong correlation with O3 ( r2 = 0.8 ) (see Fig.1) suggesting a possible link with the chemical reactions which destroy tropospheric ozone, as first suggested by Schroeder et al., 1998. Dramatically increased levels of Hg(p) and RGM were measured as Hg0 concentrations dropped to undetectable levels during the 2003 experiment. It seems reasonable to suppose that Br-compounds or another halogen-containing radical or compound formed in the period around polar sunrise, is responsible for a sudden increase in oxidation of Hg0(g) and the formation of less volatile Hg(II) compounds, which are either dry deposited to the underlying surface or condense onto pre-existing particulate matter (Lu et al., 2001; Lindberg et al., 2002). The data obtained (see Fig. 2) indicate that RGM is clearly the primary species formed during the MDE period and differently from Hg(p). In particular, from April 19th  to April 23rd  (the period covering the first major MDEs), particulate Hg and RGM showed a different trend with Hg(p) peaking on the 21st and on the 22nd when Hg0 was at its minimum concentration, but decreasing rapidly when the Hg0 concentrations again increased. The 2003 findings from Zeppelin Station suggested that different reaction pathways or reactants may be, probably, responsible for creating Hg(p) as compared to RGM during the first and major MDE. In contrast, RGM concentrations progressively increased within the first major depletion event, showing an opposite trend to Hg0 during the whole time-series concentrations. In order to analyse and interpret the mercury behaviour recorded during the experiment, daily averaged BrO Vertical Column Densities observed from the GOME satellite were used. GOME BrO distributions revealed incidence of elevated BrO column densities simultaneously with the period of low Hg0 and O3 surface levels and elevated RGM levels coincided with a period characterized by increased levels of BrO. The preliminary analysis suggests that Hg0 destruction and simultaneous RGM production, is driven by an action of atmospheric halogen chemistry in the air at Ny-Alesund involving reactive Br-compounds. In order to evaluate the role played by the transport on the surface depletions recorded during the arctic experiment and, in particular, to test whether the low Hg0 concentrations observed might be a result of transport of air masses already depleted in mercury, detailed back trajectory analyses have been carried out and compared with daily average BrO vertical column density maps from the GOME satellite. The results from this investigation regarding the first and the second depletion events clearly show that the air masses arriving at Ny-Alesund on the 22nd, which is the day with maximum depletion, arrived from north-north-east and east-north-east. The trajectories originates from the Eastern High Arctic Sea. The comparison between the back-trajectories with the GOME BrO map thus suggests that the first strong MDEs occurring at Ny-Alesund coincide with the arrival of air masses that resided most of the time over the ocean, covered with more than 40% sea ice, before reaching Zeppelin Station. The trajectory thus stated in regions with strongly elevated Br-compounds. The air masses around 400, 500 and 600 meters above sea level pass through the arctic Sea inside the boundary layer until Ny-Alesund.  These findings support the theory that the air mass with depleted mercury originated far from Ny-Alesund, before the measurement location. To examine potential influences of the meteorological factors on atmospheric reaction occurred during MDEs, Hg0 concentration-time series together with selected additional meteorological and chemical parameters were evaluated. Results obtained give clear evidence that the first depletion is associated with falling temperature and raising wind speed (T°C about –24°C and ws about 10ms-1 for a few hours) and a rapid shift in wind direction to N-NW confirming the intrusion of an Arctic air mass. The 2003 measurements thus suggest that the MDEs occurred at Ny-Alesund were caused by transport of air masses depleted in Hg0 rather than solely the result of atmospheric mercury chemistry in the air at Ny-Alesund. It is however clear that more research is needed to improve our understanding of different aspects on the mercury cycle in the polar atmosphere.

Acknowledgements

This work was funded by the United States Environmental Protection Agency (USEPA) and by the Italian National Research Council (CNR) under the cooperative agreement X4-83097301-0.

References

Hedgecock, I. M., Pirrone, N., Sprovieri, F., Pesenti, E. (2003). Atmospheric Environment; Vol. 37, pp. 41-49.

Landis, M. S., Stevens, R. K., Schaedlich, F., Prestbo, E. M. (2002). Environ. Sci. Technol. ; Vol. 36, No. 13, pp. 3000-3009.

Lindberg, S. E., Brooks, S., Lin, C.-J., Scott, K. J., Landis, M. S., Stevens, R. K., Goodsite, M., Richter A. (2002). Environ. Sci. Technol.; Vol. 36, pp. 1245-1256.

Lu, J. Y., Schroeder, W. H., Barrie, L. A., Steffen, A., Welch, H. E., Martin, K., Lockhart, W. L., Hunt, R. V., Boila, G., Richter, A. (2001). Geophys. Res. Lett.; Vol. 28, pp. 3219-3222.

# Munthe, J., Wangberg, I., Pirrone, N., Iverfeld, A., Ferrara, R., Ebinghaus, R., Feng., R., Gerdfelt, K., Keeler, G.J., Lanzillotta, E., Lindberg, S.E., Lu, J., Mamane, Y., Prestbo, E., Schmolke, S., Schroder, W.H., Sommar, J., Sprovieri, F., Stevens, R.K., Stratton, W., Tuncel, G., Urba, A. (2001) Atmos. Environ.; Vol. 35, pp. 3007-3017.

Schroeder, W. H., Anlauf, K. G., Barrie, L. A., Lu, J. Y., Steffen, A., Schneeberger, D. R., Berg, T. (1998).  Nature; Vol. 394, pp. 331-332.

Sprovieri, F., Pirrone, N., Hedgecock, I. M., Landis, M.S., Stevens, R.K. (2002). Journal of  Geophysical  Research; Vol. 107,  (D23), 4722.

Sprovieri, F., Pirrone, N., Gerdfelt, K., Sommar, J. (2003). Atmos. Environ.; Vol. 37, pp. 63-71.

Pirrone, N., Ferrara, R., Hedgecock, I. M., Kallos, G., Mamane, Y., Munte, J., Pacyna, J.M., Pytharoulis, I., Sprovieri, F., Voudouri, A., Wangberg, I. (2003) . Atmos. Environ.; Vol. 37, pp. 21-39