Microbial Ecology of Mercury Reducing Biofilms

I. Wagner-Döbler1, H. von Canstein1, A. Felske1, J. Leonhäuser1, B.V. Pauling1, W. Fehr1 and W.-D. Deckwer2

1Dept. Microbiology, National Research Institute for Biotechnology (GBF), Braunschweig, Germany

2Technical University, Braunschweig, Germany

The ancient mechanism that microbes possess for detoxification of mercury, encoded in the archetypical mer operon, has been applied to the clean-up of industrial wastewater in technical scale. A biofilm of naturally mercury resistant bacteria growing on inorganic porous carrier material forms the core of the process. The bacteria are reducing toxic ionic mercury to non toxic elemental mercury, which is accumulated in the bed. Such a bioreactor can be operated at high bed loadings (2 h-1 corresponding to a residence time of only 30 min) and high inflow Hg concentrations (up to 10 mg L-1). It was scaled up and a complete plant, including pH adjustment, automated feeding of the bacteria, and remote control was installed in a mobile container. The plant was operated on site  chloralkali electrolysis factories for 4 years continuously and shown to be efficient and robust. The new technology is low cost and environmentally friendly and might also be applicable for clean-up of soil washwater or gas scrubber solutions.

The performance of the bioreactor was stable under a wide range of wastewater parameters. It could be shown that this stability was mediated by the niche diversity in the packed bed, and by the physiological diversity of the mercury resistant strains constituting the biofilm community. A multispecies biofilm consisting of strains with different growth characteristics and mercury resistance levels was much more effective and stable than a monospecies biofilm consisting of one high performance strain. In the technical bioreactor, a succession of strains invading from the environment was observed, which were mercury resistant and therefore replaced some of the inoculum strains without detectable effect on bioreactor performance. The diversity of mercury reducing bacteria in the bioreactor was smallest at high mercury concentrations, indicating that mercury in the wastewater exerted a strong selective pressure on the bacteria. Thus, the mercury reducing biofilm community was subject to a process of adaptation and self-optimization driven by the selective pressure of mercury toxicity.

More information and pdf-files of papers under http://www.gbf.de/mercury_remediation1/index.html