Heavy Metal Contamination and Risk Assessment around

Some Abandoned Metal Mine Sites in Korea

Hyo-Taek Chon and Jin-Soo Lee

School of Civil, Urban and Geosystem Engineering, College of Engineering,

Seoul National University, Seoul 151-742, Korea, E-mail: chon@snu.ac.kr

Environmental geochemical surveys were undertaken around some abandoned metal mines (Dongil Au-Ag-Cu-Pb-Zn, Okdong Cu-Pb-Zn, Songcheon Au-Ag, Dongjung Au-Ag-Pb-Zn, Dokok Au-Ag-Cu-Pb-Zn, Hwacheon Au-Ag-Pb-Zn) to investigate the contamination levels of heavy metals and assess the risk of adverse health effects on human exposure to heavy metals influenced by past mining activities. After appropriate sample preparation, tailings, soils, crop plants and waters were analyzed for As, Cd, Cu, Pb and Zn by ICP-AES and ICP-MS.

Arsenic and other heavy metals were highly elevated in tailings from the Dongil (8,720 As mg/kg, 5.9 Cd mg/kg, 3,610 Cu mg/kg, 5,850 Pb mg/kg, 630 Zn mg/kg), the Songcheon (24,080 As mg/kg, 8.2 Cd mg/kg, 130 Cu mg/kg, 3,830 Pb mg/kg, 2,410 Zn mg/kg) and Dongjung (3,620 As mg/kg, 12.2 Cd mg/kg, 144 Cu mg/kg, 5,140 Pb mg/kg, 3,060 Zn mg/kg) mines. Also high concentrations of heavy metals except As were found in tailings from the Okdong (53.6 Cd mg/kg, 910 Cu mg/kg, 1,590 Pb mg/kg, 5,720 Zn mg/kg), the Dokok (98.2 Cd mg/kg, 2,550 Cu mg/kg, 4,200 Pb mg/kg, 18,020 Zn mg/kg) and the Hwacheon (12.4 Cd mg/kg, 580 Pb mg/kg, 1,300 Zn mg/kg) mines. These significant concentrations can impact on soils and waters around the tailing dumps.

Elevated levels of As, Cd, Cu, Pb and Zn were also found in agricultural soils from these mine areas. In particular, As concentration in agricultural soils from the Dongil, the Songcheon and the Donjung mines was higher more than the permissible level. This level is the threshold of the element concentrations in soils above which crop produced are considered as unsafe for human health. Cadmium was elevated in agricultural soils from the Okdong and the Dongjung mines, Cu from the Dongil and the Songcheon mines, and Pb from the Songcheon, the Dongjung and the Hwacheon mines. Arsenic, Cd and Zn in stream water used for drinking water in the Songcheon mine area were higher than the permissible levels (0.05 As mg/L, 0.01 Cd mg/L and 1.0 Zn mg/L) in Korea. In the Dongil mine, As in groundwater used for drinking water exceeded the permissible level (0.01 mg/L) suggested by WHO.

Risk assessment is the process of characterizing the adverse health effects of human exposure to environmental hazards. Risk compounds deriving from mine sites either constitute a toxic risk or a carcinogenic risk. Toxic risks are indicated in terms of a hazard quotient (H.Q.). H.Q. is ADD (average daily dose)/RfD (reference dose). A toxic risk exists for H.Q.>1. To arrive at the hazard index (H.I.) the ADD from three identified pathways (soil, groundwater and food (rice grain) pathways) compared to the relevant RfD obtained from the US-EPA database IRIS is summed. The hazard index of As in the Dongil, the Okdong, the Songcheon and the Hwacheon mine areas were higher value more than 1.0. In the Okdong and the Songcheon mine areas, H.I. value of Cd exceeded 1.0. These values of As and Cd were the highest in the Songcheon mine area. Therefore, toxic risk for As and Cd exist via exposure (ingestion) of contaminated soil, groundwater and rice grain in these mine areas. The cancer risk for As in stream or ground water used for drinking water from the Songcheon, the Dongil, the Dongjung and the Hwacheon mine areas was 3E-3, 5E-4, 2E-4 and 1E-4, respectively. These risk levels exceed the range required for remedial actions of 1 by 100,000 to 1,000,000.