Effect of solar radiation on mercury mobility in intertidal sediments

João Canário & Carlos Vale

National Institute for Agronomy and Fisheries Research - IPIMAR, Av. Brasilia,      1449-006 Lisbon, Portugal, Email: jcanario@ipimar.pt

Abstract: Anoxic sediments from two areas of the Tagus estuary were homogenised and distributed by two sets of 36 uncapped Petri dishes. The samples were placed on the intertidal sediments and exposed to direct solar radiation and kept in dark (control) for 6-8 hours. The decrease rates of acid volatile sulphides (abrupt in the first 3 hours) and of pyrite (linear) were the same in sediments under light and dark conditions. The total Hg concentrations were relatively constant in sediments kept in dark, but decreased from 17.6 to 7.65 nmol g-1 and from 3.45 to 1.35 nmol g-1 in the experiments I and II, respectively, in those exposed to solar radiation during the period of higher UV radiation. Similar evolutions were found in non-reactive Hg in pore waters (3.00 to 2.59 nM and 0.725 to 0.105 nM). On the contrary, reactive Hg was higher in pore waters of the sediments exposed to solar radiation and increased with the time, 424 to 845 pM and 53 to 193 pM. These results indicate that most mercury released in pore waters was photochemically reduced in a short period of time and escaped rapidly to the atmosphere. In the estuary, the transfer of Hg0 from intertidal sediments to the atmosphere is probably enhanced by during episodes of strong bottom resuspension and bioturbation.

Key words: mercury, solar radiation, intertidal sediments.

Introduction

Topmost layers of coastal sediments are frequently resuspended by waves and tidal currents and, at low tide, subsurface anoxic sediments may be temporarily exposed to the atmosphere. During these periods the sediment chemistry tends to change due to exposure to solar radiation and air, in particular, elements that are redox sensitive and those that have a high vapour pressure like mercury. Moreover, dredging operations in harbours and navigational channels cause exposure of anoxic sediments to the atmosphere. This communication reports the effect of exposing anoxic sediments to the atmosphere, under direct solar radiation and dark conditions, on the alterations of mercury concentrations in solids and pore waters. The field experiments were performed during the atmospheric exposure of intertidal sediments at two areas of the Tagus estuary with contrasting degree of contamination.

Results and discussion

Subsurface anoxic sediments were collected in two sites of the Tagus estuary with different degree of Hg contamination. In each site collected sediments were homogenised and aliquots were distributed by 36 uncapped Petri dishes. The thickness of each sediment-set was 0.5-cm. The dishes were placed on intertidal sediments at two conditions: 18 dishes in dark (covered with a black box), and 18 dishes exposed to direct solar radiation. The field experiment, run for 6-8 hours during the tidal period, was air-exposed. Air and sediment temperature as well as total UV radiation intensity were measured during these experiments. Two samples were taken each hour and analysed for (i) total Hg and Fe (CV-AAS and FAAS); (ii) Hg and Fe extracted by HCl (1M) and hydroxylammonium (0.04 M); (iii) acid volatile sulphide (AVS) and pyrite by polarography. Pore waters were also separated by centrifugation from the solids and analysed for reactive (HgR) and non-reactive (HgNR) dissolved mercury by CV-FAS.

AVS concentrations in the sediment samples decreased abruptly in the first three hours, and the gradients were independent of the light conditions (experiment I: 650 to 45 µmol g-1; Experiment II: 2.36 to 0.09 µmol g-1). At both experiments pyrite concentration decreased linearly with the time (r2 better than 0.91, p<0.05). The decreases of AVS and pyrite in the sediments indicate rapid oxidation independent of the light conditions (Figure 1). During the experiments, mercury concentrations in sediments kept in dark were relatively constant, but decreased considerably in those exposed to solar radiation. This contrasting pattern was observed in the two experiments, in spite of the differences in initial Hg concentrations (Figure 1).

Figure 1 – Evolutions of acid volatile sulphides (AVS), pyrite (mol g-1) and total mercury concentrations (nmol g-1) in sediments under light and dark conditions in Experiment I.

During the first 3 hours of Experiment I, Hg decreased abruptly from 17.6 to 10.0 nmol g-1, and then to 7.65 nmol g-1. A more gradual decrease was observed in Experiment II: from 3.45 to 1.35 nmol g-1 during 8 hours. A similar time-course evolution was also observed for both HgR and HgNR. In the two experiments HgR concentrations were higher in pore waters exposed to solar radiation than in dark. Moreover, HgR under light conditions increased with the time (424-845 pM and 53-196 pM in Experiments I and II, respectively). On the contrary, HgNR concentrations in the same conditions decreased from 3.00 to 2.59 nM in Experiment I and from 0.725 to 0.105 nM in Experiment II (Figure 2).

These field experiments, which simulate processes occurring in intertidal sediments when temporarily exposed to the atmosphere around low tide, evidence the release of Hg principally due to the effect of solar radiation. Under dark conditions only a slight increase was measured in reactive Hg in pore waters, probably due to oxidation of sulphides. When sediments remain exposed to solar radiation, total Hg concentrations decreased more than 50%, the non-reactive Hg forms in pore waters 75-90%, and reactive Hg increased drastically.

Figure 2 – Evolutions of reactive – HgR - (pM) and non-reactive – HgNR - (nM) dissolved mercury concentrations in sediments under light and dark conditions in Experiment I

The substantial decrease of Hg in sediments coincided with the period of higher UV radiation around 14:00h (GMT). Since radiation had no effect on the pyrite oxidation, other processes should induce additional mercury released to pore waters, probably the oxidation of particulate organic matter by UV radiation. The direct photoreduction from the solid sediments should not be excluded, since HgS can be photochemically reduced and semiconductor compounds (e.g., metal oxides) can promote Hg photoreduction in providing charge carriers and valence band holes (Nriagu, 1994).  The increases of reactive mercury in pore waters at the end of the Experiments I and II, under light conditions, (0.29 and 0.05 pmol g-1, respectively) can be considered negligible in comparison to the decrease in solids (10,000 and 2,100 pmol g-1, respectively). The unbalanced situations mean that most mercury released from solids to pore waters should have been photochemically reduced in short periods of time and escaped rapidly to the atmosphere. In fact, several mercury species such as complexes with OH-, HS-, Cl- and organic ligands can absorb the highly energetic UV of the solar spectrum (Fujita et al., 1973; Stromberg et al., 1991; Nriagu, 1994), and consequently be reduced in its excited state. According to Nriagu (1994) photolysis rate is rather fast, occurring in order of seconds to hours. This finding supports the rapid release of Hg from sediments observed in the experiments. Apparently, reactive mercury (HgR) in pore waters corresponds to an intermediate step.

Conclusions

These results suggest the rapid release of Hg from the solids, presumably associated with organic matter, and the destruction of Hg-complexes in pore waters due to the effect of direct solar radiation. Since increases in reactive forms do not balance decreases in solids and in non-reactive forms, volatile Hg are probably formed and evaded to the atmosphere.

Acknowledgements

This work was funded by the Portuguese Ministry of Science and Higher Education through the restructuring program MACAC, MLEM12/2000.

## References

Fujita, S., Horri, H., Taniguchi, S., (1973): Pulse photolysis of mercuric ion in aqueous solutions. Journal of Physical Chemistry 77, pp. 2868-2871.

Nriagu, J.O., (1994): Mechanistic steps in the photoreduction of mercury in natural waters. The Science of the Total Environment 154, pp. 1-8.

Stromberg, D., Stromberg, A., Wahlgren, U., (1991): Relative quantum calculations on some mercury sulfide molecules. Water, Air and Soil Pollution 56, pp. 681-695.